ASxxxx Assemblers

and

ASLINK Relocating Linker

Version   2.10

April 1999

Page 2

P R E F A C E

The  ASxxxx  assemblers  were  written following the style of

several cross assemblers found in the Digital Equipment Corpora-

tion  Users  Society  (DECUS)  distribution of the C programming

language.  The DECUS code was provided with no documentation  as

to  the  input  syntax  or the output format.  Study of the code

revealed that the unknown author of the code  had  attempted  to

formulate  an assembler with attributes similiar to those of the

PDP-11 MACRO assembler (without macro's).  The  incomplete  code

from  the  DECUS C distribution has been largely rewritten, only

the program structure, and C source  file  organization  remains

relatively  unchanged.   However, I wish to thank the author for

his contribution to this set of assemblers.

The  ASLINK  program was written as a companion to the ASxxxx

assemblers, its design and implementation was not  derived  from

any other work.

The  ASxxxx  assemblers  and the ASLINK relocating linker are

placed in the Public Domain.   Publication  or  distribution  of

these programs for non-commercial use is hereby granted with the

stipulation that the  copyright  notice  be  included  with  all

copies.

I  would  greatly  appreciate  receiving  the  details of any

changes, additions, or errors pertaining to these  programs  and

will  attempt  to  incorporate  any  fixes  or  generally useful

changes in a future update to these programs.

Alan R.  Baldwin

Kent State University

Physics Department

Kent, Ohio 44242

U.S.A.

http://shop-pdp.kent.edu/ashtml/asxxxx.htm

baldwin@shop-pdp.kent.edu

tel:  (330) 672 2531

fax:  (330) 672 2959

Page 3

C O N T R I B U T O R S

Thanks  to  Marko  Makela  for his contribution of the AS6500

cross assembler.

Marko Makela

Sillitie 10 A

01480 Vantaa

Finland

Internet: Marko.Makela@Helsinki.Fi

EARN/BitNet: msmakela@finuh

Thanks  to  John  Hartman  for his contribution of the AS8051

cross assembler and updates to the ASxxxx and ASLINK internals.

John L. Hartman

jhartman@compuserve.com

http://ourworld.compuserve.com/homepages/jhartman/

Thanks  to  G.   Osborn  for his contributions to LKS19.C and

LKIHX.C.

G. Osborn

gary@s-4.com

Page 4

ASxxxx Cross Assemblers, Version 2.10, April 1999

Submitted by Alan R.  Baldwin,

Kent State University, Kent, Ohio

Operating System:  TSX+, RT-11, MS-DOS, Windows 3.x/95/98, Linux

or other supporting K&R C.

Source Langauge:  C

Abstract:

The  ASxxxx  assemblers are a series of microprocessor assem-

blers written in the C programming  language.   This  collection

contains cross assemblers for the 6800(6802/6808), 6801(hd6303),

6804,  6805,  68HC08,  6809,  68HC11,  68HC12,   68HC16,   8051,

8085(8080),  z80(hd64180),  H8/3xx, and 6500 series microproces-

sors.  Each  assembler  has  a  device  specific  section  which

includes:   (1)  device description, byte order, and file exten-

sion information, (2) a table of assembler  general  directives,

special directives, assembler mnemonics and associated operation

codes, (3) machine  specific  code  for  processing  the  device

mnemonics, addressing modes, and special directives.

The assemblers have a common device independent section which

handles the details of file input/output, symbol  table  genera-

tion,  program/data  areas,  expression  analysis, and assembler

directive processing.

The  assemblers  provide  the following features:  (1) alpha-

betized, formatted symbol table listings, (2) relocatable object

modules, (3) global symbols for linking object modules, (4) con-

ditional assembly directives, (5) reusable  local  symbols,  and

(6) include-file processing.

The  companion program ASLINK is a relocating linker perform-

ing the following functions:  (1) bind multiple  object  modules

into  a  single  memory  image,  (2) resolve inter-module symbol

references,  (3)  resolve  undefined  symbols   from   specified

librarys of object modules, (4) process absolute, relative, con-

catenated, and overlay attributes in data and program  sections,

(5)  perform  byte and word program-counter relative (pc or pcr)

addressing calculations, (6) define absolute  symbol  values  at

link  time, (7) define absolute area base address values at link

time, (8) produce Intel Hex or Motorola  S19  output  file,  (9)

produce  a  map  of the linked memory image, and (10) update the

ASxxxx assembler listing files  with  the  absolute  linked  ad-

dresses and data.

The assemblers and linker have been tested using DECUS C with

TSX+ and RT-11, GCC 2.7.2 with LINUX, Symantec  C/C++  V6.1/V7.2

and  DJGPP  (GCC 2.8.1) with MS-DOS/Windows 3.x/95/98.  Complete

source code and documentation for the assemblers and  linker  is

included  with  the  distribution.   Additionally, test code for

each assembler and several microprocessor  monitors  (  ASSIST05

for  the 6805, MONDEB and ASSIST09 for the 6809, and BUFFALO 2.5

for the 6811) are included as working examples of use  of  these

assemblers.

CHAPTER 1  THE ASSEMBLER                                     1-1

1.1     THE ASXXXX ASSEMBLERS                              1-1

1.1.1     Assembly Pass 1                                  1-2

1.1.2     Assembly Pass 2                                  1-2

1.1.3     Assembly Pass 3                                  1-2

1.2     SOURCE PROGRAM FORMAT                              1-3

1.2.1     Statement Format                                 1-3

1.2.1.1     Label Field                                    1-3

1.2.1.2     Operator Field                                 1-5

1.2.1.3     Operand Field                                  1-5

1.2.1.4     Comment Field                                  1-6

1.3     SYMBOLS AND EXPRESSIONS                            1-6

1.3.1     Character Set                                    1-6

1.3.2     User-Defined Symbols                            1-10

1.3.3     Local Symbols                                   1-11

1.3.4     Current Location Counter                        1-12

1.3.5     Numbers                                         1-14

1.3.6     Terms                                           1-14

1.3.7     Expressions                                     1-15

1.4     GENERAL ASSEMBLER DIRECTIVES                      1-16

1.4.1     .module Directive                               1-16

1.4.2     .title Directive                                1-17

1.4.3     .sbttl Directive                                1-17

1.4.4     .page Directive                                 1-17

1.4.5     .byte and .db Directives                        1-17

1.4.6     .word and .dw Directives                        1-18

1.4.7     .blkb, .blkw, and .ds Directives                1-18

1.4.8     .ascii Directive                                1-18

1.4.9     .ascis Directive                                1-19

1.4.10    .asciz Directive                                1-19

1.4.11    .radix Directive                                1-20

1.4.12    .even Directive                                 1-20

1.4.13    .odd Directive                                  1-20

1.4.14    .area Directive                                 1-21

1.4.15    .org Directive                                  1-22

1.4.16    .globl Directive                                1-23

1.4.17    .if, .else, and .endif Directives               1-23

1.4.18    .include Directive                              1-24

1.4.19    .setdp Directive                                1-25

1.5     INVOKING ASXXXX                                   1-27

1.6     ERRORS                                            1-28

1.7     LISTING FILE                                      1-29

1.8     SYMBOL TABLE FILE                                 1-31

1.9     OBJECT FILE                                       1-31

CHAPTER 2  THE LINKER                                        2-1

2.1     ASLINK RELOCATING LINKER                           2-1

2.2     INVOKING ASLINK                                    2-2

2.3     LIBRARY PATH(S) AND FILE(S)                        2-3

2.4     ASLINK PROCESSING                                  2-4

2.5     LINKER INPUT FORMAT                                2-6

2.5.1     Object Module Format                             2-6

2.5.2     Header Line                                      2-6

2.5.3     Module Line                                      2-7

2.5.4     Symbol Line                                      2-7

2.5.5     Area Line                                        2-7

Page ii

2.5.6     T Line                                           2-7

2.5.7     R Line                                           2-8

2.5.8     P Line                                           2-8

2.6     LINKER ERROR MESSAGES                              2-9

2.7     INTEL HEX OUTPUT FORMAT                           2-11

2.8     MOTORLA S1-S9 OUTPUT FORMAT                       2-12

CHAPTER 3  BUILDING ASXXXX AND ASLINK                        3-1

3.1     BUILDING AN ASSEMBLER                              3-1

3.2     BUILDING ASLINK                                    3-2

APPENDIX A  AS6800 ASSEMBLER                                 A-1

A.1     6800 REGISTER SET                                  A-1

A.2     6800 INSTRUCTION SET                               A-1

A.2.1     Inherent Instructions                            A-2

A.2.2     Branch Instructions                              A-2

A.2.3     Single Operand Instructions                      A-3

A.2.4     Double Operand Instructions                      A-4

A.2.5     Jump and Jump to Subroutine Instructions         A-4

A.2.6     Long Register Instructions                       A-5

APPENDIX B  AS6801 ASSEMBLER                                 B-1

B.1     .hd6303 DIRECTIVE                                  B-1

B.2     6801 REGISTER SET                                  B-1

B.3     6801 INSTRUCTION SET                               B-1

B.3.1     Inherent Instructions                            B-2

B.3.2     Branch Instructions                              B-2

B.3.3     Single Operand Instructions                      B-3

B.3.4     Double Operand Instructions                      B-4

B.3.5     Jump and Jump to Subroutine Instructions         B-5

B.3.6     Long Register Instructions                       B-5

B.3.7     6303 Specific Instructions                       B-5

APPENDIX C  AS6804 ASSEMBLER                                 C-1

C.1     6804 REGISTER SET                                  C-1

C.2     6804 INSTRUCTION SET                               C-1

C.2.1     Inherent Instructions                            C-2

C.2.2     Branch Instructions                              C-2

C.2.3     Single Operand Instructions                      C-2

C.2.4     Jump and Jump to Subroutine Instructions         C-2

C.2.5     Bit Test Instructions                            C-2

C.2.6     Load Immediate data Instruction                  C-3

C.2.7     6804 Derived Instructions                        C-3

APPENDIX D  AS6805 ASSEMBLER                                 D-1

D.1     6805 REGISTER SET                                  D-1

D.2     6805 INSTRUCTION SET                               D-1

D.2.1     Control Instructions                             D-2

D.2.2     Bit Manipulation Instructions                    D-2

D.2.3     Branch Instructions                              D-2

D.2.4     Read-Modify-Write Instructions                   D-3

D.2.5     Register\Memory Instructions                     D-3

Page iii

D.2.6     Jump and Jump to Subroutine Instructions         D-4

APPENDIX E  AS6808 ASSEMBLER                                 E-1

E.1     68HC08 REGISTER SET                                E-1

E.2     68HC08 INSTRUCTION SET                             E-1

E.2.1     Control Instructions                             E-2

E.2.2     Bit Manipulation Instructions                    E-2

E.2.3     Branch Instructions                              E-3

E.2.4     Complex Branch Instructions                      E-3

E.2.5     Read-Modify-Write Instructions                   E-4

E.2.6     Register\Memory Instructions                     E-5

E.2.7     Double Operand Move Instruction                  E-5

E.2.8     16-Bit <H:X> Index Register Instructions         E-5

E.2.9     Jump and Jump to Subroutine Instructions         E-5

APPENDIX F  AS6809 ASSEMBLER                                 F-1

F.1     6809 REGISTER SET                                  F-1

F.2     6809 INSTRUCTION SET                               F-1

F.2.1     Inherent Instructions                            F-3

F.2.2     Short Branch Instructions                        F-3

F.2.3     Long Branch Instructions                         F-3

F.2.4     Single Operand Instructions                      F-4

F.2.5     Double Operand Instructions                      F-5

F.2.6     D-register Instructions                          F-5

F.2.7     Index/Stack Register Instructions                F-5

F.2.8     Jump and Jump to Subroutine Instructions         F-6

F.2.9     Register - Register Instructions                 F-6

F.2.10    Condition Code Register Instructions             F-6

F.2.11    6800 Compatibility Instructions                  F-6

APPENDIX G  AS6811 ASSEMBLER                                 G-1

G.1     68HC11 REGISTER SET                                G-1

G.2     68HC11 INSTRUCTION SET                             G-1

G.2.1     Inherent Instructions                            G-2

G.2.2     Branch Instructions                              G-2

G.2.3     Single Operand Instructions                      G-3

G.2.4     Double Operand Instructions                      G-4

G.2.5     Bit Manupulation Instructions                    G-4

G.2.6     Jump and Jump to Subroutine Instructions         G-5

G.2.7     Long Register Instructions                       G-5

APPENDIX H  AS6812 ASSEMBLER                                 H-1

H.1     68HC12 REGISTER SET                                H-1

H.2     68HC12 INSTRUCTION SET                             H-1

H.2.1     Inherent Instructions                            H-3

H.2.2     Short Branch Instructions                        H-3

H.2.3     Long Branch Instructions                         H-3

H.2.4     Branch on Decrement, Test, or Increment          H-4

H.2.5     Bit Clear and Set Instructions                   H-4

H.2.6     Branch on Bit Clear or Set                       H-4

H.2.7     Single Operand Instructions                      H-5

H.2.8     Double Operand Instructions                      H-6

Page iv

H.2.9     Move Instructions                                H-6

H.2.10    D-register Instructions                          H-6

H.2.11    Index/Stack Register Instructions                H-7

H.2.12    Jump and Jump/Call to Subroutine

Instructions                                     H-7

H.2.13    Other Special Instructions                       H-7

H.2.14    Register - Register Instructions                 H-7

H.2.15    Condition Code Register Instructions             H-7

H.2.16    M68HC11 Compatibility Mode Instructions          H-8

APPENDIX I  AS6816 ASSEMBLER                                 I-1

I.1     68HC16 REGISTER SET                                I-1

I.2     68HC16 INSTRUCTION SET                             I-1

I.2.1     Inherent Instructions                            I-2

I.2.2     Push/Pull Multiple Register Instructions         I-3

I.2.3     Short Branch Instructions                        I-3

I.2.4     Long Branch Instructions                         I-3

I.2.5     Bit Manipulation Instructions                    I-3

I.2.6     Single Operand Instructions                      I-4

I.2.7     Double Operand Instructions                      I-5

I.2.8     Index/Stack Register Instructions                I-5

I.2.9     Jump and Jump to Subroutine Instructions         I-6

I.2.10    Condition Code Register Instructions             I-6

I.2.11    Multiply and Accumulate Instructions             I-6

APPENDIX J  ASH8 ASSEMBLER                                   J-1

J.1     H8/3XX REGISTER SET                                J-1

J.2     H8/3XX INSTRUCTION SET                             J-1

J.2.1     Inherent Instructions                            J-2

J.2.2     Branch Instructions                              J-2

J.2.3     Single Operand Instructions                      J-3

J.2.4     Double Operand Instructions                      J-4

J.2.5     Mov Instructions                                 J-5

J.2.6     Bit Manipulation Instructions                    J-6

J.2.7     Extended Bit Manipulation Instructions           J-7

J.2.8     Condition Code Instructions                      J-7

J.2.9     Other Instructions                               J-8

J.2.10    Jump and Jump to Subroutine Instructions         J-8

APPENDIX K  AS8051 ASSEMBLER                                 K-1

K.1     ACKNOWLEDGMENT                                     K-1

K.2     8051 REGISTER SET                                  K-1

K.3     8051 INSTRUCTION SET                               K-2

K.3.1     Inherent Instructions                            K-2

K.3.2     Move Instructions                                K-3

K.3.3     Single Operand Instructions                      K-3

K.3.4     Two Operand Instructions                         K-4

K.3.5     Call and Return Instructions                     K-4

K.3.6     Jump Instructions                                K-4

K.3.7     Predefined Symbols:  SFR Map                     K-5

K.3.8     Predefined Symbols:  SFR Bit Addresses           K-6

K.3.9     Predefined Symbols:  Control Bits                K-7

Page v

APPENDIX L  AS8085 ASSEMBLER                                 L-1

L.1     8085 REGISTER SET                                  L-1

L.2     8085 INSTRUCTION SET                               L-1

L.2.1     Inherent Instructions                            L-2

L.2.2     Register/Memory/Immediate Instructions           L-2

L.2.3     Call and Return Instructions                     L-2

L.2.4     Jump Instructions                                L-2

L.2.5     Input/Output/Reset Instructions                  L-3

L.2.6     Move Instructions                                L-3

L.2.7     Other Instructions                               L-3

APPENDIX M  ASZ80 ASSEMBLER                                  M-1

M.1     .hd64 DIRECTIVE                                    M-1

M.2     Z80 REGISTER SET AND CONDITIONS                    M-1

M.3     Z80 INSTRUCTION SET                                M-2

M.3.1     Inherent Instructions                            M-3

M.3.2     Implicit Operand Instructions                    M-3

M.3.3     Load Instruction                                 M-4

M.3.4     Call/Return Instructions                         M-4

M.3.5     Jump and Jump to Subroutine Instructions         M-4

M.3.6     Bit Manipulation Instructions                    M-5

M.3.7     Interrupt Mode and Reset Instructions            M-5

M.3.8     Input and Output Instructions                    M-5

M.3.9     Register Pair Instructions                       M-5

M.3.10    HD64180 Specific Instructions                    M-6

APPENDIX N  AS6500 ASSEMBLER                                 N-1

N.1     ACKNOWLEDGMENT                                     N-1

N.2     6500 REGISTER SET                                  N-2

N.3     6500 INSTRUCTION SET                               N-2

N.3.1     Processor Specific Directives                    N-3

N.3.2     65xx Core Inherent Instructions                  N-3

N.3.3     65xx Core Branch Instructions                    N-3

N.3.4     65xx Core Single Operand Instructions            N-3

N.3.5     65xx Core Double Operand Instructions            N-4

N.3.6     65xx Core Jump and Jump to Subroutine

Instructions                                     N-4

N.3.7     65xx Core Miscellaneous X and Y Register

Instructions                                     N-4

N.3.8     65F11 and 65F12 Specific Instructions            N-5

N.3.9     65C00/21 and 65C29 Specific Instructions         N-5

N.3.10    65C02, 65C102, and 65C112 Specific

Instructions                                     N-6

CHAPTER 1

THE ASSEMBLER

1.1  THE ASXXXX ASSEMBLERS

The  ASxxxx  assemblers are a series of microprocessor assem-

blers written in the C programming language.  Each assembler has

a device specific section which includes:

1.  device  description, byte order, and file extension in-

formation

2.  a  table  of  the assembler general directives, special

device directives, assembler mnemonics  and  associated

operation codes

3.  machine specific code for processing the device mnemon-

ics, addressing modes, and special directives

The device specific information is detailed in the appendices.

The assemblers have a common device independent section which

handles the details of file input/output, symbol  table  genera-

tion,  program/data  areas,  expression  analysis, and assembler

directive processing.

The assemblers provide the following features:

1.  Command string control of assembly functions

2.  Alphabetized, formatted symbol table listing

3.  Relocatable object modules

4.  Global symbols for linking object modules

5.  Conditional assembly directives

THE ASSEMBLER                                           PAGE 1-2

THE ASXXXX ASSEMBLERS

6.  Program sectioning directives

ASxxxx assembles one or more source files into a single relo-

catable ascii object file.  The output of the ASxxxx  assemblers

consists of an ascii relocatable object file(*.rel), an assembly

listing file(*.lst), and a symbol file(*.sym).

1.1.1  Assembly Pass 1

During  pass  1, ASxxxx opens all source files and performs a

rudimenatry assembly of each source statement.  During this pro-

cess  all symbol tables are built, program sections defined, and

number of bytes for each assembled source line is estimated.

At the end of pass 1 all undefined symbols may be made global

(external) using the ASxxxx switch -g, otherwise undefined  sym-

bols will be flagged as errors during succeeding passes.

1.1.2  Assembly Pass 2

During  pass  2  the ASxxxx assembler resolves forward refer-

ences and determines the number  of  bytes  for  each  assembled

line.   The  number  of bytes used by a particular assembler in-

struction may depend upon the addressing mode, whether  the  in-

struction allows multiple forms based upon the relative distance

to the addressed location, or other factors.   Pass  2  resolves

these cases and determines the address of all symbols.

1.1.3  Assembly Pass 3

Pass 3 by the assembler generates the listing file, the relo-

catable output file, and the symbol tables.  Also during pass  3

the errors will be reported.

The  relocatable object file is an ascii file containing sym-

bol references and definitions, program  area  definitions,  and

the  relocatable assembled code, the linker ASLINK will use this

information to generate an absolute load file (Motorola or Intel

formats).

THE ASSEMBLER                                           PAGE 1-3

SOURCE PROGRAM FORMAT

1.2  SOURCE PROGRAM FORMAT

1.2.1  Statement Format

A source program is composed of assembly-language statements.

Each statement must be completed on one line.  A line  may  con-

tain a maximum of 128 characters, longer lines are truncated and

lost.

An  ASxxxx  assembler  statement  may  have  as  many as four

fields.  These fields are identified by their order  within  the

statement  and/or  by separating characters between fields.  The

general format of the ASxxxx statement is:

[label:]  Operator        Operand         [;Comment(s)]

The  label and comment fields are optional.  The operator and

operand fields are interdependent.  The operator field may be an

assembler  directive or an assembly mnemonic.  The operand field

may be optional or required as defined in  the  context  of  the

operator.

ASxxxx  interprets  and  processes source statements one at a

time.  Each statement causes a particular operation to  be  per-

formed.

1.2.1.1  Label Field  -

A  label is a user-defined symbol which is assigned the value

of the current location counter and entered into  the  user  de-

fined  symbol  table.   The  current location counter is used by

ASxxxx to assign memory addresses to the source  program  state-

ments as they are encountered during the assembly process.  Thus

a label is a means  of  symbolically  referring  to  a  specific

statement.

When  a program section is absolute, the value of the current

location counter is absolute;  its value references an  absolute

memory  address.   Similarly, when a program section is relocat-

able, the value of the current location counter is  relocatable.

A  relocation  bias  calculated at link time is added to the ap-

parent value of the current location counter  to  establish  its

effective  absolute  address  at  execution time.  (The user can

also force the linker to relocate sections defined as  absolute.

This may be required under special circumstances.)

If  present,  a  label  must  be  the first field in a source

statement and must be terminated by a colon (:).   For  example,

THE ASSEMBLER                                           PAGE 1-4

SOURCE PROGRAM FORMAT

if  the  value  of  the  current  location  counter  is absolute

01F0(H), the statement:

abcd:     nop

assigns  the  value  01F0(H) to the label abcd.  If the location

counter value were relocatable, the final value of abcd would be

01F0(H)+K, where K represents the relocation bias of the program

section, as calculated by the linker at link time.

More  than  one label may appear within a single label field.

Each label so specified is assigned the same address value.  For

example,  if  the  value  of  the  current  location  counter is

1FF0(H), the multiple labels in the following statement are each

assigned the value 1FF0(H):

abcd:     aq:     $abc:   nop

Multiple labels may also appear on successive lines.  For ex-

ample, the statements

abcd:

aq:

$abc:     nop

likewise  cause  the  same value to be assigned to all three la-

bels.

A  double  colon  (::)  defines the label as a global symbol.

For example, the statement

abcd::    nop

establishes the label abcd as a global symbol.  The distinguish-

ing attribute of a global symbol is that it  can  be  referenced

from  within an object module other than the module in which the

symbol is defined.  References to this label  in  other  modules

are  resolved when the modules are linked as a composite execut-

able image.

The legal characters for defining labels are:

A through Z

a through z

0 through 9

. (Period)

$ (Dollar sign)

_ (underscore)

A  label  may  be  any  length,  however  only  the  first 79

characters are significant and, therefore must be  unique  among

all   labels  in  the  source  program  (not  necessarily  among

THE ASSEMBLER                                           PAGE 1-5

SOURCE PROGRAM FORMAT

separately compiled modules).  An error code(s) (m or p) will be

generated  in the assembly listing if the first 79 characters in

two or more labels are the same.  The m code is  caused  by  the

redeclaration  of  the symbol or its reference by another state-

ment.  The p code is generated because the symbols  location  is

changing on each pass through the source file.

The  label  must  not  start with the characters 0-9, as this

designates a local symbol with special attributes described in a

later section.

The  label  must  not  start  with  the  sequence $$, as this

represents the temporary radix 16 for constants.

1.2.1.2  Operator Field  -

The  operator field specifies the action to be performed.  It

may consist of an instruction mnemonic (op code) or an assembler

directive.

When  the  operator is an instruction mnemonic, a machine in-

struction is generated and the assembler evaluates the addresses

of  the operands which follow.  When the operator is a directive

ASxxxx performs certain control actions or processing operations

during assembly of the source program.

Leading  and  trailing  spaces  or tabs in the operator field

have no significance;  such characters serve  only  to  separate

the operator field from the preceeding and following fields.

An operator is terminated by a space, tab or end of line.

1.2.1.3  Operand Field  -

When  the  operator is an instruction mnemonic (op code), the

operand  field  contains  program  variables  that  are  to   be

evaluated/manipulated by the operator.

Operands  may  be  expressions  or  symbols, depending on the

operator.  Multiple expressions used in the operand  fields  may

be  separated  by a comma.  An operand should be preceeded by an

operator field;  if it is not, the statement will give an  error

(q  or  o).   All  operands  following instruction mnemonics are

treated as expressions.

The operand field is terminated by a semicolon when the field

is followed  by  a  comment.   For  example,  in  the  following

statement:

label:    lda     abcd,x          ;Comment field

THE ASSEMBLER                                           PAGE 1-6

SOURCE PROGRAM FORMAT

the  tab  between lda and abcd terminates the operator field and

defines the beginning of the operand field;  a  comma  separates

the operands abcd and x;  and a semicolon terminates the operand

field and defines the beginning of the comment field.   When  no

comment  field  follows,  the operand field is terminated by the

end of the source line.

1.2.1.4  Comment Field  -

The comment field begins with a semicolon and extends through

the end of the line.  This field is optional and may contain any

7-bit ascii character except null.

Comments  do not affect assembly processing or program execu-

tion.

1.3  SYMBOLS AND EXPRESSIONS

This  section  describes the generic components of the ASxxxx

assemblers:  the character set, the conventions observed in con-

structing  symbols,  and  the use of numbers, operators, and ex-

pressions.

1.3.1  Character Set

The following characters are legal in ASxxxx source programs:

1.  The  letters  A  through Z.  Both upper- and lower-case

letters are acceptable.  The  assemblers,  by  default,

are  not  case  sensitive,  i.e.  ABCD and abcd are the

same symbols.  (The assemblers can be made case  sensi-

tive by using the -z command line option.)

2.  The digits 0 through 9

3.  The  characters . (period), $ (dollar sign), and _ (un-

derscore).

4.  The special characters listed in Tables 1 through 6.

Tables  1  through  6  describe  the various ASxxxx label and

field terminators, assignment operators, operand separators, as-

sembly, unary, binary, and radix operators.

THE ASSEMBLER                                           PAGE 1-7

SYMBOLS AND EXPRESSIONS

Table 1         Label Terminators and Assignment Operators

----------------------------------------------------------------

:   Colon               Label terminator.

::  Double colon        Label  Terminator;   defines the

label as a global label.

=   Equal sign          Direct assignment operator.

==  Double equal        Direct assignment operator;

sign                defines  the  symbol as a global

symbol.

----------------------------------------------------------------

Table 2         Field Terminators and Operand Separators

----------------------------------------------------------------

Tab                 Item or field terminator.

Space               Item or field terminator.

,   Comma               Operand field separator.

;   Semicolon           Comment field indicator.

----------------------------------------------------------------

Table 3         Assembler Operators

----------------------------------------------------------------

#   Number sign         Immediate expression indicator.

.   Period              Current location counter.

(   Left parenthesis    Expression delimiter.

)   Right parenthesis   Expression delimeter.

----------------------------------------------------------------

THE ASSEMBLER                                           PAGE 1-8

SYMBOLS AND EXPRESSIONS

Table 4         Unary Operators

----------------------------------------------------------------

<   Left bracket        <FEDC   Produces  the lower byte

value of the expression.

(DC)

>   Right bracket       >FEDC   Produces  the upper byte

value of the expression.

(FE)

+   Plus sign           +A      Positive value of A

-   Minus sign          -A      Produces   the  negative

(2's complement) of A.

~   Tilde               ~A      Produces the 1's comple-

ment of A.

'   Single quote        'D      Produces  the  value  of

the character D.

"   Double quote        "AB     Produces the double byte

value for AB.

\   Backslash           '\n     Unix style characters

\b, \f, \n, \r, \t

or '\001   or octal byte values.

----------------------------------------------------------------

THE ASSEMBLER                                           PAGE 1-9

SYMBOLS AND EXPRESSIONS

Table 5         Binary Operators

----------------------------------------------------------------

<<  Double          0800 << 4   Produces the 4 bit

Left bracket                left-shifted   value  of

0800.  (8000)

>>  Double          0800 >> 4   Produces the 4 bit

Right bracket               right-shifted  value  of

0800.  (0080)

+   Plus sign       A + B       Arithmetic      Addition

operator.

-   Minus sign      A - B       Arithmetic   Subtraction

operator.

*   Asterisk        A * B       Arithmetic   Multiplica-

tion operator.   (signed

16-bit)

/   Slash           A / B       Arithmetic      Division

operator.        (signed

16-bit quotient)

&   Ampersand       A & B       Logical AND operator.

|   Bar             A | B       Logical OR operator.

%   Percent sign    A % B       Modulus operator.

(16-bit value)

^   Up arrow or     A ^ B       EXCLUSIVE OR operator.

circumflex

----------------------------------------------------------------

THE ASSEMBLER                                          PAGE 1-10

SYMBOLS AND EXPRESSIONS

Table 6         Temporary Radix Operators

----------------------------------------------------------------

$%,   0b, 0B            Binary radix operator.

$&,   0o, 0O, 0q, 0Q    Octal radix operator.

$#,   0d, 0D            Decimal radix operator.

$$,   0h, 0H, 0x, 0X    Hexidecimal radix operator.

Potential  ambiguities arising from the use of 0b and 0d

as temporary radix operators may be circumvented by pre-

ceding  all  non-prefixed  hexidecimal  numbers with 00.

Leading 0's are required in any  case  where  the  first

hexidecimal  digit is abcdef as the assembler will treat

the letter sequence as a label.

----------------------------------------------------------------

1.3.2  User-Defined Symbols

User-defined  symbols are those symbols that are equated to a

specific value through a direct assignment statement  or  appear

as  labels.  These symbols are added to the User Symbol Table as

they are encountered during assembly.

The following rules govern the creation of user-defined symbols:

1.  Symbols  can  be  composed  of alphanumeric characters,

dollar signs ($),  periods  (.),  and  underscores  (_)

only.

2.  The  first  character  of a symbol must not be a number

(except in the case of local symbols).

3.  The  first 79 characters of a symbol must be unique.  A

symbol  can  be  written  with  more  than   79   legal

characters,  but the 80th and subsequent characters are

ignored.

4.  Spaces and Tabs must not be embedded within a symbol.

THE ASSEMBLER                                          PAGE 1-11

SYMBOLS AND EXPRESSIONS

1.3.3  Local Symbols

Local  symbols are specially formatted symbols used as labels

within a block of coding that has been delimited as a local sym-

bol  block.   Local  symbols  are  of  the form n$, where n is a

decimal integer from 0 to 255,  inclusive.   Examples  of  local

symbols are:

1$

27$

138$

244$

The  range  of  a local symbol block consists of those state-

ments between two normally constructed  symbolic  labels.   Note

that a statement of the form:

ALPHA = EXPRESSION

is a direct assignment statement but does not create a label and

thus does not delimit the range of a local symbol block.

Note that the range of a local symbol block may extend across

program areas.

Local symbols provide a convenient means of generating labels

for branch instructions and other such references  within  local

symbol  blocks.   Using local symbols reduces the possibility of

symbols with multiple definitions appearing within a  user  pro-

gram.   In  addition,  the  use  of local symbols differentiates

entry-point labels from local labels, since local labels  cannot

be referenced from outside their respective local symbol blocks.

Thus, local symbols of the same name can appear in  other  local

symbol blocks without conflict.  Local symbols require less sym-

bol table space than normal symbols.  Their use is recommended.

The  use of the same local symbol within a local symbol block

will generate one or both of the m or p errors.

THE ASSEMBLER                                          PAGE 1-12

SYMBOLS AND EXPRESSIONS

Example of local symbols:

a:      ldx     #atable ;get table address

lda     #0d48   ;table length

1$:     clr     ,x+     ;clear

deca

bne     1$

b:      ldx     #btable ;get table address

lda     #0d48   ;table length

1$:     clr     ,x+     ;clear

deca

bne     1$

1.3.4  Current Location Counter

The  period  (.) is the symbol for the current location coun-

ter.  When used in the operand  field  of  an  instruction,  the

period   represents  the  address  of  the  first  byte  of  the

instruction:

AS:     ldx     #.      ;The period (.) refers to

;the address of the ldx

;instruction.

When  used  in  the  operand field of an ASxxxx directive, it

represents the address of the current byte or word:

QK = 0

.word   0xFFFE,.+4,QK   ;The operand .+4 in the .word

;directive represents a value

;stored in the second of the

;three words during assembly.

If  we  assume  the  current  value of the program counter is

0H0200, then during assembly, ASxxxx  reserves  three  words  of

storage  starting  at  location 0H0200.  The first value, a hex-

idecimal constant FFFE, will be stored at location 0H0200.   The

second  value  represented  by  .+4  will  be stored at location

0H0202, its value will be 0H0206 ( = 0H0202  +  4).   The  third

value  defined  by  the  symbol  QK  will  be placed at location

0H0204.

At the beginning of each assembly pass, ASxxxx resets the lo-

cation counter.  Normally, consecutive memory locations are  as-

signed  to  each  byte  of  object code generated.  However, the

value of the location counter can be changed  through  a  direct

assignment statement of the following form:

THE ASSEMBLER                                          PAGE 1-13

SYMBOLS AND EXPRESSIONS

. = . + expression

The  new  location  counter can only be specified relative to

the current location counter.  Neglecting to specify the current

program  counter  along with the expression on the right side of

the assignment operator will generate the (.) error.   (Absolute

program areas may use the .org directive to specify the absolute

location of the current program counter.)

The following coding illustrates the use of the current location

counter:

.area   CODE1   (ABS)   ;program area CODE1

;is ABSOLUTE

.org    0H100           ;set location to

;0H100 absolute

num1:   ldx     #.+0H10         ;The label num1 has

;the value 0H100.

;X is loaded with

;0H100 + 0H10

.org    0H130           ;location counter

;set to 0H130

num2:   ldy     #.              ;The label num2 has

;the value 0H130.

;Y is loaded with

;value 0H130.

.area   CODE2   (REL)   ;program area CODE2

;is RELOCATABLE

. = . + 0H20            ;Set location counter

;to relocatable 0H20 of

;the program section.

num3:   .word   0               ;The label num3 has

;the value

;of relocatable 0H20.

. = . + 0H40            ;will reserve 0H40

;bytes of storage as will

.blkb   0H40            ;or

.blkw   0H20

The  .blkb  and .blkw directives are the preferred methods of

allocating space.

THE ASSEMBLER                                          PAGE 1-14

SYMBOLS AND EXPRESSIONS

1.3.5  Numbers

ASxxxx  assumes that all numbers in the source program are to

be interpreted in decimal radix unless otherwise specified.  The

.radix  directive  may  be used to specify the default as octal,

decimal, or hexidecimal.  Individual numbers can  be  designated

as  binary, octal, decimal, or hexidecimal through the temporary

radix prefixes shown in table 6.

Negative  numbers  must be preceeded by a minus sign;  ASxxxx

translates such numbers into two's  complement  form.   Positive

numbers may (but need not) be preceeded by a plus sign.

Numbers are always considered to be absolute values, therefor

they are never relocatable.

1.3.6  Terms

A  term is a component of an expression and may be one of the

following:

1.  A number.

2.  A symbol:

1.  A  period (.) specified in an expression causes the

current location counter to be used.

2.  A User-defined symbol.

3.  An undefined symbol is assigned a value of zero and

inserted in the User-Defined symbol table as an un-

defined symbol.

3.  A single quote followed by a single ascii character, or

a double quote followed by two ascii characters.

4.  An  expression enclosed in parenthesis.  Any expression

so enclosed is evaluated and reduced to a  single  term

before  the remainder of the expression in which it ap-

pears is evaluated.  Parenthesis, for example,  may  be

used  to  alter the left-to-right evaluation of expres-

sions, (as in A*B+C versus A*(B+C)), or to apply a  un-

ary operator to an entire expression (as in -(A+B)).

5.  A unary operator followed by a symbol or number.

THE ASSEMBLER                                          PAGE 1-15

SYMBOLS AND EXPRESSIONS

1.3.7  Expressions

Expressions  are  combinations  of  terms  joined together by

binary operators.  Expressions reduce to a  16-bit  value.   The

evaluation  of  an  expression includes the determination of its

attributes.  A resultant expression value may be  one  of  three

types  (as  described  later in this section):  relocatable, ab-

solute, and external.

Expressions are evaluate with an operand hierarchy as follows:

*       /       %       multiplication,

division, and

modulus first.

+       -               addition and

subtraction second.

<<      >>              left shift and

right shift third.

^                       exclusive or fourth.

&                       logical and fifth.

|                       logical or last

except that unary operators take precedence over binary

operators.

A  missing  or  illegal  operator  terminates  the expression

analysis, causing error codes (o) and/or  (q)  to  be  generated

depending upon the context of the expression itself.

At assembly time the value of an external (global) expression

is equal to the value of the absolute part of  that  expression.

For  example,  the expression external+4, where 'external' is an

external symbol, has the value of 4.  This expression,  however,

when  evaluated  at link time takes on the resolved value of the

symbol 'external', plus 4.

Expressions,  when  evaluated  by  ASxxxx,  are  one of three

types:  relocatable, absolute, or external.  The following  dis-

tinctions are important:

1.  An  expression is relocatable if its value is fixed re-

lative to the base address of the program area in which

it appears;  it will have an offset value added at link

time.  Terms that contain labels defined in relocatable

program   areas   will   have   a   relocatable  value;

THE ASSEMBLER                                          PAGE 1-16

SYMBOLS AND EXPRESSIONS

similarly, a period (.) in a relocatable program  area,

representing  the value of the current program location

counter, will also have a relocatable value.

2.  An  expression  is  absolute if its value is fixed.  An

expression whose terms are numbers and ascii characters

will  reduce  to  an absolute value.  A relocatable ex-

pression or term minus a relocatable term,  where  both

elements  being  evaluated  belong  to the same program

area, is an absolute expression.  This is because every

term  in  a  program area has the same relocation bias.

When one term is subtracted from the other the  reloca-

tion bias is zero.

3.  An  expression is external (or global) if it contains a

single global reference (plus or minus an absolute  ex-

pression  value) that is not defined within the current

program.  Thus, an external  expression  is  only  par-

tially  defined following assembly and must be resolved

at link time.

1.4  GENERAL ASSEMBLER DIRECTIVES

An  ASxxxx  directive  is placed in the operator field of the

source line.  Only one directive is  allowed  per  source  line.

Each  directive  may  have  a blank operand field or one or more

operands.  Legal operands differ with each directive.

1.4.1  .module Directive

Format:

.module string

The .module directive causes the string to be included in the

assemblers output file as an identifier for this particular  ob-

ject  module.   The  string  may  be  from 1 to 79 characters in

length.  Only one identifier is allowed  per  assembled  module.

The  main use of this directive is to allow the linker to report

a modules' use of undefined symbols.  At link time all undefined

symbols  are  reported  and  the  modules  referencing  them are

listed.

THE ASSEMBLER                                          PAGE 1-17

GENERAL ASSEMBLER DIRECTIVES

1.4.2  .title Directive

Format:

.title  string

The .title directive provides a character string to be placed

on the second line of each page during listing.

1.4.3  .sbttl Directive

Format:

.sbttl  string

The .sbttl directive provides a character string to be placed

on the third line of each page during listing.

1.4.4  .page Directive

Format:

.page

The .page directive causes a page ejection with a new heading

to be printed.  The new page occurs after the next line  of  the

source  program is processed, this allows an immediately follow-

ing .sbttl directive to appear  on  the  new  page.   The  .page

source  line will not appear in the file listing.  Paging may be

disabled by invoking the -p directive.

1.4.5  .byte and .db Directives

Format:

.byte   exp             ;Stores the binary value

.db     exp             ;of the expression in the

;next byte.

.byte   exp1,exp2,expn  ;Stores the binary values

.db     exp1,exp2,expn  ;of the list of expressions

;in successive bytes.

where:  exp,    represent expressions that will be

exp1,   truncated to 8-bits of data.

.       Each expression will be calculated

.       as a 16-bit word expression,

.       the high-order byte will be truncated.

.       Multiple expressions must be

THE ASSEMBLER                                          PAGE 1-18

GENERAL ASSEMBLER DIRECTIVES

expn    separated by commas.

The  .byte  or .db directives are used to generate successive

bytes of binary data in the object module.

1.4.6  .word and .dw Directives

Format:

.word   exp             ;Stores the binary value

.dw     exp             ;of the expression in

;the next word.

.word   exp1,exp2,expn  ;Stores the binary values

.dw     exp1,exp2,expn  ;of the list of expressions

;in successive words.

where:  exp,    represent expressions that will occupy two

exp1,   bytes of data. Each expression will be

.       calculated as a 16-bit word expression.

.       Multiple expressions must be

expn    separated by commas.

The  .word  or .dw directives are used to generate successive

words of binary data in the object module.

1.4.7  .blkb, .blkw, and .ds Directives

Format:

.blkb   N       ;reserve N bytes of space

.blkw   N       ;reserve N words of space

.ds     N       ;reserve N bytes of space

The  .blkb  and .ds directives reserve byte blocks in the ob-

ject module;  the .blkw directive reserves word blocks.

1.4.8  .ascii Directive

Format:

.ascii  /string/

where:  string  is a string of printable ascii characters.

/  /    represent   the  delimiting  characters.   These

delimiters   may   be   any   paired    printing

characters,  as  long  as the characters are not

contained within  the  string  itself.   If  the

THE ASSEMBLER                                          PAGE 1-19

GENERAL ASSEMBLER DIRECTIVES

delimiting  characters  do not match, the .ascii

directive will give the (q) error.

The  .ascii  directive  places  one binary byte of data for each

character in the string into the object module.

1.4.9  .ascis Directive

Format:

.ascis  /string/

where:  string  is a string of printable ascii characters.

/  /    represent   the  delimiting  characters.   These

delimiters   may   be   any   paired    printing

characters,  as  long  as the characters are not

contained within  the  string  itself.   If  the

delimiting  characters  do not match, the .ascis

directive will give the (q) error.

The  .ascis  directive  places  one binary byte of data for each

character in the  string  into  the  object  module.   The  last

character in the string will have the high order bit set.

1.4.10  .asciz Directive

Format:

.asciz  /string/

where:  string  is a string of printable ascii characters.

/  /    represent   the  delimiting  characters.   These

delimiters   may   be   any   paired    printing

characters,  as  long  as the characters are not

contained within  the  string  itself.   If  the

delimiting  characters  do not match, the .asciz

directive will give the (q) error.

The  .asciz  directive  places  one binary byte of data for each

character in the string into the object module.   Following  all

the  character  data  a  zero  byte is inserted to terminate the

character string.

THE ASSEMBLER                                          PAGE 1-20

GENERAL ASSEMBLER DIRECTIVES

1.4.11  .radix Directive

Format:

.radix  character

where:  character  represents  a single character specifying the

default radix to be used for succeeding numbers.

The character may be any one of the following:

B,b     Binary

O,o     Octal

Q,q

D,d     Decimal

'blank'

H,h     Hexidecimal

X,x

1.4.12  .even Directive

Format:

.even

The .even directive ensures that the current location counter

contains an even boundary value by adding 1 if the current loca-

tion is odd.

1.4.13  .odd Directive

Format:

.odd

The  .odd directive ensures that the current location counter

contains an odd boundary value by adding one if the current  lo-

cation is even.

THE ASSEMBLER                                          PAGE 1-21

GENERAL ASSEMBLER DIRECTIVES

1.4.14  .area Directive

Format:

.area   name    [(options)]

where:  name    represents the symbolic name of the program sec-

tion.   This  name  may  be  the  same  as   any

user-defined  symbol  as  the area names are in-

dependent of all symbols and labels.

options specify the type of program or data area:

ABS     absolute (automatically invokes OVR)

REL     relocatable

OVR     overlay

CON     concatenate

PAG     paged area

The .area directive provides a means of defining and separat-

ing multiple programming and data sections.   The  name  is  the

area  label used by the assembler and the linker to collect code

from various separately assembled modules into one section.  The

name may be from 1 to 79 characters in length.

The options are specified within parenthesis and separated by

commas as shown in the following example:

.area  TEST  (REL,CON)  ;This section is relocatable

;and concatenated with other

;sections of this program area.

.area  DATA  (REL,OVR)  ;This section is relocatable

;and overlays other sections

;of this program area.

.area  SYS   (ABS,OVR)  ;(CON not allowed with ABS)

;This section is defined as

;absolute. Absolute sections

;are always overlayed with

;other sections of this program

;area.

.area  PAGE  (PAG)      ;This is a paged section. The

;section must be on a 256 byte

;boundary and its length is

;checked by the linker to be

;no larger than 256 bytes.

;This is useful for direct page

;areas.

THE ASSEMBLER                                          PAGE 1-22

GENERAL ASSEMBLER DIRECTIVES

The  default  area type is REL|CON;  i.e.  a relocatable sec-

tion which is concatenated with other sections of code with  the

same area name.  The ABS option indicates an absolute area.  The

OVR and CON options indicate if program  sections  of  the  same

name  will overlay each other (start at the same location) or be

concatenated with each other (appended to each other).

Multiple  invocations  of  the  .area directive with the same

name must specify the same options or leave  the  options  field

blank,  this  defaults  to  the previously specified options for

this program area.

The   ASxxxx   assemblers   automatically  provide  two  program

sections:

'.  .ABS.'      This dumby section contains all absolute

symbols and their values.

'_CODE'         This  is  the default program/data area.

This program area is of type (REL,CON).

The  ASxxxx  assemblers  also automatically generate two symbols

for each program area:

's_<area>'      This is the starting address of the pro-

gram area.

indent  -16  'l_<area>'      This is the

length of the program area.

The .area names and options are never case sensitive.

1.4.15  .org Directive

Format:

.org    exp

where:  exp     is  an absolute expression that becomes the cur-

rent location counter.

The  .org directive is valid only in an absolute program section

and will give a (q) error if used in a relocatable program area.

The  .org  directive specifies that the current location counter

is to become the specified absolute value.

THE ASSEMBLER                                          PAGE 1-23

GENERAL ASSEMBLER DIRECTIVES

1.4.16  .globl Directive

Format:

.globl  sym1,sym2,...,symn

where:  sym1,           represent legal symbolic names. When

sym2,...        When multiple symbols are specified,

symn            they are separated by commas.

A  .globl directive may also have a label field and/or a com-

ment field.

The  .globl directive is provided to define (and thus provide

linkage to) symbols not  otherwise  defined  as  global  symbols

within  a  module.   In  defining  global  symbols the directive

.globl J is similar to:

J == expression or J::

Because  object  modules  are linked by global symbols, these

symbols are vital to a program.  All internal symbols  appearing

within  a  given program must be defined at the end of pass 1 or

they will be considered undefined.  The assembly directive  (-g)

can  be  be  invoked to make all undefined symbols global at the

end of pass 1.

1.4.17  .if, .else, and .endif Directives

Format:

.if     expr

.                       ;}

.                       ;} range of true condition

.                       ;}

.else

.                       ;}

.                       ;} range of false condition

.                       ;}

.endif

The  conditional  assembly directives allow you to include or

exclude blocks of source code during the assembly process, based

on the evaluation of the condition test.

The  range of true condition will be processed if the expres-

sion 'expr' is not zero (i.e.  true) and the range of false con-

dition  will  be processed if the expression 'expr' is zero (i.e

false).  The range of true condition is optional as is the .else

directive  and  the range of false condition.  The following are

all valid .if/.else/.endif constructions:

THE ASSEMBLER                                          PAGE 1-24

GENERAL ASSEMBLER DIRECTIVES

.if     A-4             ;evaluate A-4

.byte   1,2             ;insert bytes if A-4 is

.endif                  ;not zero

.if     K+3             ;evaluate K+3

.else

.byte   3,4             ;insert bytes if K+3

.endif                  ;is zero

.if     J&3             ;evaluate J masked by 3

.byte   12              ;insert this byte if J&3

.else                   ;is not zero

.byte   13              ;insert this byte if J&3

.endif                  ;is zero

The .if/.else/.endif directives may be nested upto 10 levels.

The  .page  directive  is  processed within a false condition

range to allow extended textual information to  be  incorporated

in  the  source  program  with  out  the need to use the comment

delimiter (;):

.if     0

.page

This text will be bypassed during assembly

but appear in the listing file.

.

.

.

.endif

1.4.18  .include Directive

Format:

.include        string

where:  string  represents  a  delimited string that is the file

specification of an ASxxxx source file.

The .include directive is used to insert a source file within

the source file currently being assembled.  When this  directive

is encountered, an implicit .page directive is issued.  When the

end of the specified source file is reached, an  implicit  .page

directive is issued and input continues from the previous source

file.  The maximum nesting level of source files specified by  a

.include directive is five.

THE ASSEMBLER                                          PAGE 1-25

GENERAL ASSEMBLER DIRECTIVES

The  total  number  of separately specified .include files is

unlimited as each .include file is opened and then closed during

each pass made by the assembler.

The  default  directory  path,  if none is specified, for any

.include file is the directory path of the  current  file.   For

example:   if  the  current  source file, D:\proj\file1.asm, in-

cludes  a  file  specified   as   "include1"   then   the   file

D:\proj\include1.asm is opened.

1.4.19  .setdp Directive

Format:

.setdp [base [,area]]

The  set  direct  page  directive has a common format in all the

AS68xx assemblers.  The .setdp directive is used to  inform  the

assembler  of  the current direct page region and the offset ad-

dress within the selected area.  The normal  invocation  methods

are:

.area   DIRECT  (PAG)

.setdp

or

.setdp  0,DIRECT

for  all  the  68xx microprocessors (the 6804 has only the paged

ram area).  The commands specify that the direct page is in area

DIRECT and its offset address is 0 (the only valid value for all

but the 6809 microprocessor).  Be sure to place the DIRECT  area

at address 0 during linking.  When the base address and area are

not specified, then zero and the current area are the  defaults.

If  a  .setdp directive is not issued the assembler defaults the

direct page to the area "_CODE" at offset 0.

The  assembler  verifies  that  any  local variable used in a

direct variable reference is located in this area.  Local  vari-

able  and  constant value direct access addresses are checked to

be within the address range from 0 to 255.

External direct references are assumed by the assembler to be

in the correct area and have valid  offsets.   The  linker  will

check all direct page relocations to verify that they are within

the correct area.

The  6809  microprocessor  allows the selection of the direct

page to be on any 256 byte boundary by loading  the  appropriate

THE ASSEMBLER                                          PAGE 1-26

GENERAL ASSEMBLER DIRECTIVES

value  into the dp register.  Typically one would like to select

the page boundary at link time, one method follows:

.area   DIRECT  (PAG)   ; define the direct page

.setdp

.

.

.

.area   PROGRAM

.

ldd     #DIRECT         ; load the direct page register

tfr     a,dp            ; for access to the direct page

At  link  time specify the base and global equates to locate the

direct page:

-b DIRECT = 0x1000

-g DIRECT = 0x1000

Both  the  area address and offset value must be specified (area

and variable names are independent).   The  linker  will  verify

that  the  relocated  direct page accesses are within the direct

page.

The  preceeding  sequence  could  be repeated for multiple paged

areas, however an alternate method is to define a non-paged area

and use the .setdp directive to specify the offset value:

.area   DIRECT          ; define non-paged area

.

.

.

.area   PROGRAM

.

.setdp  0,DIRECT        ; direct page area

ldd     #DIRECT         ; load the direct page register

tfr     a,dp            ; for access to the direct page

.

.

.setdp  0x100,DIRECT    ; direct page area

ldd     #DIRECT+0x100   ; load the direct page register

tfr     a,dp            ; for access to the direct page

The  linker  will  verify that subsequent direct page references

are in the specified area and offset address range.  It  is  the

programmers responsibility to load the dp register with the cor-

rect page segment  corresponding  to  the  .setdp  base  address

specified.

For  those  cases  where a single piece of code must access a

defined data structure within a direct page and there  are  many

pages,  define  a  dumby  direct page linked at address 0.  This

dumby page is used only to define  the  variable  labels.   Then

THE ASSEMBLER                                          PAGE 1-27

GENERAL ASSEMBLER DIRECTIVES

load  the dp register with the real base address but donot use a

.setdp directive.  This method is equivalent to indexed address-

ing,  where the dp register is the index register and the direct

addressing is the offset.

1.5  INVOKING ASXXXX

The ASxxxx assemblers are command line oriented.  Most sytems

require the option(s) and file(s) arguments to follow the ASxxxx

assembler name:

as6809 [-dqxgalopswzff] file1 [file2 file3 ...  file6]

Some  systems  may  request the arguments after the assembler is

started at a system specific prompt:

as6809

argv:  [-dqxgalopswzff] file1 [file2 file3 ...  file6]

The options are:

d       decimal listing

q       octal   listing

x       hex     listing (default)

The listing radix affects the

.lst, .rel, and .sym files.

g       undefined symbols made global

a       all user symbols made global

l       create list   output file1.lst

o       create object output file1.rel

s       create symbol output file1.sym

p       disable listing pagination

w       wide listing format for symbol table

z       enable case sensitivity for symbols

relocatable reference flagging:

f       by  `   in the listing file

ff      by mode in the listing file

The file name for the .lst, .rel, and .sym files is the first

file name specified in the command line.  All output  files  are

ascii  text  files which may be edited, copied, etc.  The output

THE ASSEMBLER                                          PAGE 1-28

INVOKING ASXXXX

files are the concatenation of all the input files, if files are

to  be  assembled  independently  invoke  the assembler for each

file.

The  .rel  file contains a radix directive so that the linker

will use the proper conversion for this file.  Linked files  may

have different radices.

If  the list (l) option is specified without the symbol table

(s) option, the symbol table is placed at the end of the listing

file.

1.6  ERRORS

The  ASxxxx assemblers provide limited diagnostic error codes

during the assembly process, these errors will be noted  in  the

listing file and printed on the stderr device.

The assembler reports the errors on the stderr device as

?ASxxxx-Error-<*> in line nnn of filename

where  * is the error code, nnn is the line number, and filename

is the source/include file.

The errors are:

(.)   This  error  is caused by an absolute direct assign-

ment of the current location counter

. = expression (incorrect)

rather than the correct

. = . + expression

(a)   Indicates  a machine specific addressing or address-

ing mode error.

(b)   Indicates a direct page boundary error.

(d)   Indicates a direct page addressing error.

(i)   Caused  by  an  .include file error or an .if/.endif

mismatch.

(m)   Multiple  definitions  of  the  same label, multiple

.module directives, or multiple  conflicting  attri-

butes in an .area directive.

(o)   Directive  or  mnemonic error or the use of the .org

directive in a relocatable area.

THE ASSEMBLER                                          PAGE 1-29

ERRORS

(p)   Phase error:  label location changing between passes

2 and 3.  Normally caused by having  more  than  one

level of forward referencing.

(q)   Questionable syntax:  missing or improper operators,

terminators, or delimiters.

(r)   Relocation  error:   logic  operation attempted on a

relocatable term, addition of two relocatable terms,

subtraction  of two relocatable terms not within the

same programming area or external symbols.

(u)   Undefined symbol encountered during assembly.

(z)   Divide by 0 or Modulus by 0 error:  result is 0.

1.7  LISTING FILE

The  (-l) option produces an ascii output listing file.  Each

page of output contains a four line header:

1.  The ASxxxx program name and page number

2.  Title from a .title directive (if any)

3.  Subtitle from a .sbttl directive (if any)

4.  Blank line

Each succeeding line contains five fields:

1.  Error field (first three characters of line)

2.  Current location counter

3.  Generated code in byte format

4.  Source text line number

5.  Source text

The error field may contain upto 2 error flags indicating any

errors encountered while assembling this line of source code.

THE ASSEMBLER                                          PAGE 1-30

LISTING FILE

The  current  location counter field displays the 16-bit pro-

gram position.  This field will be in the selected radix.

The generated code follows the program location.  The listing

radix determines the number of bytes that will be  displayed  in

this field.  Hexidecimal listing allows six bytes of data within

the field, decimal and octal allow four bytes within the  field.

If more than one field of data is generated from the assembly of

a single line of source code, then the data field is repeated on

successive lines.

The source text line number is printed in decimal and is fol-

lowed by the source text.

Two  special  cases  will  disable  the  listing of a line of

source text:

1.  Source line with a .page directive is never listed.

2.  Source  line  with  a  .include  file  directive is not

listed unless the .include file cannot be opened.

Two  data  field  options  are  available to flag those bytes

which will be relocated by the linker.   If  the  -f  option  is

specified  then  each  byte to be relocated will be preceeded by

the '`' character.  If the -ff option  is  specified  then  each

byte  to  be relocated will be preceeded by one of the following

characters:

1.  *   paged relocation

2.  u   low  byte of unsigned word or unsigned byte

3.  v   high byte of unsigned word

4.  p   PCR low  byte of word relocation or PCR byte

5.  q   PCR high byte of word relocation

6.  r   low  byte relocation or byte relocation

7.  s   high byte relocation

THE ASSEMBLER                                          PAGE 1-31

SYMBOL TABLE FILE

1.8  SYMBOL TABLE FILE

The symbol table has two parts:

1.  The alphabetically sorted list of symbols and/or labels

defined or referenced in the source program.

2.  A  list of the program areas defined during assembly of

the source program.

The sorted list of symbols and/or labels contains the follow-

ing information:

1.  Program  area  number (none if absolute value or exter-

nal)

2.  The symbol or label

3.  Directly assigned symbol is denoted with an (=) sign

4.  The  value of a symbol, location of a label relative to

the program area base address (=0), or a ****  indicat-

ing the symbol or label is undefined.

5.  The  characters:   G - global, R - relocatable, and X -

external.

The list of program areas provides the correspondence between

the program area numbers and the defined program areas, the size

of the program areas, and the area flags (attributes).

1.9  OBJECT FILE

The  object  file is an ascii file containing the information

needed by the linker to bind multiple object modules into a com-

plete  loadable  memory  image.   The object module contains the

following designators:

[XDQ][HL]

X       Hexidecimal radix

D       Decimal radix

Q       Octal radix

H       Most significant byte first

L       Least significant byte first

H       Header

THE ASSEMBLER                                          PAGE 1-32

OBJECT FILE

M       Module

A       Area

S       Symbol

T       Object code

R       Relocation information

P       Paging information

Refer to the linker for a detailed description of each of the

designators and the format of the information contained  in  the

object file.

CHAPTER 2

THE LINKER

2.1  ASLINK RELOCATING LINKER

ASLINK is the companion linker for the ASxxxx assemblers.

The  program ASLINK is a general relocating linker performing

the following functions:

1.  Bind multiple object modules into a single memory image

2.  Resolve inter-module symbol references

3.  Combine  code  belonging to the same area from multiple

object files into a single contiguous memory region

4.  Search and import object module libraries for undefined

global variables

5.  Perform   byte   and   word  program  counter  relative

(pc or pcr) addressing calculations

6.  Define absolute symbol values at link time

7.  Define absolute area base address values at link time

8.  Produce Intel Hex or Motorola S19 output file

9.  Produce a map of the linked memory image

10.  Produce  an updated listing file with the relocated ad-

dresses and data

THE LINKER                                              PAGE 2-2

INVOKING ASLINK

2.2  INVOKING ASLINK

Most  sytems require the options to be entered on the command

line:

aslink -[options] -[option arg] files

Some  systems  may  request the arguments after the linker is

started at a system specific prompt:

aslink

argv:  -[options] -[option arg] files

The allowed linker commands are:

1.  -c         ASlink >> prompt mode.

The  ASlink >>  prompt  mode reads linker commands from

stdin.

2.  -f  file   Command file mode.

The  command file mode imports linker commands from the

specified file (extension must be  .lnk),  imported  -c

and  -f  commands  are ignored.  If the directory path,

for a file to be linked, is not specified in  the  com-

mand  file  then  the  path  defaults  to the .lnk file

directory path.

3.  -p/-n   enable/disable echoing commands to stdout.

4.  -i/-s   Intel Hex (file.ihx) or Motorola S19 (file.s19)

image output file.

5.  -z      Specifies that symbol names are case sensitive.

6.  -m      Generate a map file (file.map).  This file con-

tains a list of the symbols (by area) with absolute ad-

dresses,  sizes  of  linked  areas,  and  other linking

information.

7.  -w      Specifies  that  a  wide listing format be used

for the map file.

8.  -xdq    Specifies  the  number  radix  for the map file

(Hexidecimal, Decimal, or Octal).

9.  -u      Generate  an  updated  listing  file (file.rst)

derived from the relocated addresses and data from  the

linker.

10.  file    File(s) to be linked.  Files may be on the same

line as the above options or on a separate line(s)  one

THE LINKER                                              PAGE 2-3

INVOKING ASLINK

file  per line or multiple files separated by spaces or

tabs.

11.  -b  area=expression (one definition per line)

This  specifies  an area base address where the expres-

sion may contain constants and/or defined symbols  from

the linked files.

12.  -g  symbol=expression (one definition per line)

This  specifies  the value for the symbol where the ex-

pression may contain constants and/or  defined  symbols

from the linked files.

13.  -k  library directory path

(one  definition  per line) This specifies one possible

path to an object library.  More than one path  is  al-

lowed.

14.  -l  library file specification

(one  definition  per  line)  This specifies a possible

library file.  More than one file is allowed.

15.  -e      or null line, terminates input to the linker.

2.3  LIBRARY PATH(S) AND FILE(S)

The process of resolving undefined symbols after scanning the

input object  files  includes  the  scanning  of  object  module

libraries.   The  linker will search through all combinations of

the library path specifications (input by the -k option) and the

library  file  specifications (input by the -l option) that lead

to an existing library file.  Each library file contains a  list

(one  file  per  line)  of  modules  included in this particular

library.  Each existing object module is scanned for a match  to

the undefined symbol.  The first module containing the symbol is

then linked with the previous modules to resolve the symbol  de-

finition.   The  library  object  modules are rescanned until no

more symbols can be resolved.   The  scanning  algorithm  allows

resolution  of  back references.  No errors are reported for non

existant library files or object modules.

The  library  file  specification may be formed in one of two

ways:

1.  If  the  library  file  contained an absolute path/file

specification  then  this  is   the   object   module's

path/file.

(i.e.  C:\...  or C:/...)

THE LINKER                                              PAGE 2-4

LIBRARY PATH(S) AND FILE(S)

2.  If  the  library  file  contains  a  relative path/file

specification then the concatenation of  the  path  and

this  file  specification  becomes  the object module's

path/file.

(i.e.  \...  or /...)

As  an example, assume there exists a library file termio.lib

in the syslib directory specifying the following object modules:

\6809\io_disk        first object module

d:\special\io_comm   second object module

and the following parameters were specified to the linker:

-k c:\iosystem\    the first path

-k c:\syslib\      the second path

-l termio          the first library file

-l io              the second library file (no such file)

The  linker  will attempt to use the following object modules to

resolve any undefined symbols:

c:\syslib\6809\io_disk.rel     (concatenated path/file)

d:\special\io_comm.rel         (absolute path/file)

all  other path(s)/file(s) don't exist.  (No errors are reported

for non existant path(s)/file(s).)

2.4  ASLINK PROCESSING

The  linker  processes  the  files  in  the  order  they  are

presented.  The first pass through the input files  is  used  to

define  all  program  areas, the section area sizes, and symbols

defined or referenced.  Undefined symbols will initiate a search

of any specified library file(s) and the importing of the module

containing the symbol definition.  After the first pass  the  -b

(area  base  address) definitions, if any, are processed and the

areas linked.

The  area  linking proceeds by first examining the area types

ABS, CON, REL, OVR and PAG.  Absolute areas (ABS) from  separate

object modules are always overlayed and have been assembled at a

specific address, these are not normally relocated (if a -b com-

mand  is  used  on an absolute area the area will be relocated).

Relative areas (normally defined as REL|CON) have a base address

of  0x0000  as read from the object files, the -b command speci-

fies the beginning address of the area.  All subsequent relative

areas  will  be  concatenated  with  proceeding  relative areas.

THE LINKER                                              PAGE 2-5

ASLINK PROCESSING

Where specific ordering is desired, the first linker input  file

should  have  the area definitions in the desired order.  At the

completion of the area linking all area  addresses  and  lengths

have  been determined.  The areas of type PAG are verified to be

on a 256 byte boundary and that the length does not  exceed  256

bytes.  Any errors are noted on stderr and in the map file.

Next  the  global symbol definitions (-g option), if any, are

processed.  The symbol definitions have been delayed until  this

point because the absolute addresses of all internal symbols are

known and can be used in the expression calculations.

Before  continuing  with the linking process the symbol table

is scanned to determine if any symbols have been referenced  but

not defined.  Undefined symbols are listed on the stderr device.

if a .module directive was included in the  assembled  file  the

module  making  the reference to this undefined variable will be

printed.

Constants  defined  as global in more than one module will be

flagged as multiple definitions if their values are not  identi-

cal.

After  the  preceeding  processes are complete the linker may

output a map file (-m option).  This file provides the following

information:

1.  Global symbol values and label absolute addresses

2.  Defined areas and there lengths

3.  Remaining undefined symbols

4.  List of modules linked

5.  List of library modules linked

6.  List of -b and -g definitions

The final step of the linking process is performed during the

second pass of the input files.  As the xxx.rel files  are  read

the code is relocated by substituting the physical addresses for

the referenced symbols and areas and may be output in  Intel  or

Motorola  formats.   The  number of files linked and symbols de-

fined/referenced is limited by the processor space available  to

build the area/symbol lists.  If the -u option is specified then

the listing files  (file.lst)  associated  with  the  relocation

files  (file.rel)  are  scanned  and  used  to create a new file

THE LINKER                                              PAGE 2-6

ASLINK PROCESSING

(file.rst) which has all addresses and data relocated  to  their

final values.

2.5  LINKER INPUT FORMAT

The  linkers'  input  object file is an ascii file containing

the information needed by the linker  to  bind  multiple  object

modules into a complete loadable memory image.

The object module contains the following designators:

[XDQ][HL]

X       Hexidecimal radix

D       Decimal radix

Q       Octal radix

H       Most significant byte first

L       Least significant byte first

H       Header

M       Module

A       Area

S       Symbol

T       Object code

R       Relocation information

P       Paging information

2.5.1  Object Module Format

The  first  line  of  an object module contains the [XDQ][HL]

format specifier (i.e.  XH indicates  a  hexidecimal  file  with

most significant byte first) for the following designators.

2.5.2  Header Line

H aa areas gg global symbols

The  header  line  specifies  the number of areas(aa) and the

number of global symbols(gg) defined or referenced in  this  ob-

ject module segment.

THE LINKER                                              PAGE 2-7

LINKER INPUT FORMAT

2.5.3  Module Line

M name

The  module  line  specifies  the module name from which this

header segment was assembled.  The module line will  not  appear

if the .module directive was not used in the source program.

2.5.4  Symbol Line

S string Defnnnn

or

S string Refnnnn

The  symbol line defines (Def) or references (Ref) the symbol

'string' with the value nnnn.  The defined value is relative  to

the  current area base address.  References to constants and ex-

ternal global symbols will always appear before the  first  area

definition.  References to external symbols will have a value of

zero.

2.5.5  Area Line

A label size ss flags ff

The  area  line  defines the area label, the size (ss) of the

area in bytes, and the area flags (ff).  The area flags  specify

the ABS, REL, CON, OVR, and PAG parameters:

OVR/CON  (0x04/0x00 i.e.  bit position 2)

ABS/REL  (0x08/0x00 i.e.  bit position 3)

PAG      (0x10 i.e.  bit position 4)

2.5.6  T Line

T xx xx nn nn nn nn nn ...

The  T  line contains the assembled code output by the assem-

bler with xx xx being the offset address from the  current  area

base address and nn being the assembled instructions and data in

byte format.

THE LINKER                                              PAGE 2-8

LINKER INPUT FORMAT

2.5.7  R Line

R 0 0 nn nn n1 n2 xx xx ...

The R line provides the relocation information to the linker.

The nn nn value is the current area index, i.e.  which area  the

current  values  were  assembled.  Relocation information is en-

coded in groups of 4 bytes:

1.  n1  is  the  relocation mode and object format, for the

adhoc extension modes refer to asxxxx.h or aslink.h

1.  bit 0  word(0x00)/byte(0x01)

2.  bit 1  relocatable area(0x00)/symbol(0x02)

3.  bit 2  normal(0x00)/PC relative(0x04) relocation

4.  bit 3  1-byte(0x00)/2-byte(0x08)  object format for

byte data

5.  bit 4  signed(0x00)/unsigned(0x10) byte data

6.  bit 5  normal(0x00)/page '0'(0x20) reference

7.  bit 6  normal(0x00)/page 'nnn'(0x40) reference

8.  bit 7  LSB  byte(0x00)/MSB  byte(0x80)  with 2-byte

mode

2.  n2  is  a byte index into the corresponding (i.e.  pre-

ceeding) T line data (i.e.  a pointer to the data to be

updated  by  the  relocation).   The T line data may be

1-byte or  2-byte  byte  data  format  or  2-byte  word

format.

3.  xx xx  is the area/symbol index for the area/symbol be-

ing referenced.  the corresponding area/symbol is found

in the header area/symbol lists.

The groups of 4 bytes are repeated for each item requiring relo-

cation in the preceeding T line.

2.5.8  P Line

P 0 0 nn nn n1 n2 xx xx

The  P  line provides the paging information to the linker as

specified by a .setdp directive.  The format of  the  relocation

information is identical to that of the R line.  The correspond-

ing T line has the following information:

T xx xx aa aa bb bb

Where  aa aa is the area reference number which specifies the

selected page area and bb bb is the base address  of  the  page.

bb bb will require relocation processing if the 'n1 n2 xx xx' is

specified in the P line.  The linker will verify that  the  base

THE LINKER                                              PAGE 2-9

LINKER INPUT FORMAT

address is on a 256 byte boundary and that the page length of an

area defined with the PAG type is not larger than 256 bytes.

The  linker  defaults any direct page references to the first

area defined in the input REL file.  All ASxxxx assemblers  will

specify the _CODE area first, making this the default page area.

2.6  LINKER ERROR MESSAGES

The linker provides detailed error messages allowing the pro-

grammer to quickly find the errant code.   As  the  linker  com-

pletes  pass 1  over  the  input  file(s)  it  reports  any page

boundary or page length errors as follows:

?ASlink-Warning-Paged Area PAGE0 Boundary Error

and/or

?ASlink-Warning-Paged Area PAGE0 Length Error

where PAGE0 is the paged area.

During  Pass  two the linker reads the T, R, and P lines per-

forming the necessary relocations and  outputting  the  absolute

code.  Various errors may be reported during this process

The P line processing can produce only one possible error:

?ASlink-Warning-Page Definition Boundary Error

file        module      pgarea      pgoffset

PgDef  t6809l      t6809l      PAGE0       0001

The error message specifies the file and module where the .setdp

direct was issued and indicates  the  page  area  and  the  page

offset value determined after relocation.

The R line processing produces various errors:

?ASlink-Warning-Byte PCR relocation error for symbol  bra2

file        module      area        offset

Refby  t6809l      t6809l      TEST        00FE

Defin  tconst      tconst      .  .ABS.    0080

?ASlink-Warning-Unsigned Byte error for symbol  two56

file        module      area        offset

Refby  t6800l      t6800l      DIRECT      0015

Defin  tconst      tconst      .  .ABS.    0100

THE LINKER                                             PAGE 2-10

LINKER ERROR MESSAGES

?ASlink-Warning-Page0 relocation error for symbol  ltwo56

file        module      area        offset

Refby  t6800l      t6800l      DIRECT      000D

Defin  tconst      tconst      DIRECT      0100

?ASlink-Warning-Page Mode relocation error for symbol  two56

file        module      area        offset

Refby  t6809l      t6809l      DIRECT      0005

Defin  tconst      tconst      .  .ABS.    0100

?ASlink-Warning-Page Mode relocation error

file        module      area        offset

Refby  t           Pagetest    PROGRAM     0006

Defin  t           Pagetest    DIRECT      0100

These  error messages specify the file, module, area, and offset

within the area of the code  referencing  (Refby)  and  defining

(Defin) the symbol.  If the symbol is defined in the same module

as the reference the linker is unable to report the symbol name.

The  assembler  listing file(s) should be examined at the offset

from the specified area to located the offending code.

The errors are:

1.  The  byte PCR error is caused by exceeding the pc rela-

tive byte branch range.

2.  The Unsigned byte error indicates an indexing value was

negative or larger than 255.

3.  The  Page0  error is generated if the direct page vari-

able is not in the page0 range of 0 to 255.

4.  The page mode error is generated if the direct variable

is not within the current direct page (6809).

THE LINKER                                             Page 2-11

INTEL HEX OUTPUT FORMAT

2.7  INTEL HEX OUTPUT FORMAT

Record Mark Field    -  This  field  signifies  the  start  of a

record, and consists of an  ascii  colon

(:).

Record Length Field  -  This   field   consists   of  two  ascii

characters which indicate the number  of

data   bytes   in   this   record.   The

characters are the result of  converting

the  number  of  bytes  in binary to two

ascii characters, high digit first.   An

End  of  File  record contains two ascii

zeros in this field.

Load Address Field   -  This  field  consists  of the four ascii

characters which result from  converting

the  the  binary value of the address in

which to begin loading this record.  The

order is as follows:

High digit of high byte of address.

Low digit of high byte of address.

High digit of low byte of address.

Low digit of low byte of address.

In an End of File record this field con-

sists of either four ascii zeros or  the

program  entry  address.   Currently the

entry address option is not supported.

Record Type Field    -  This  field  identifies the record type,

which is either 0 for data records or  1

for  an End of File record.  It consists

of two ascii characters, with  the  high

digit of the record type first, followed

by the low digit of the record type.

Data Field           -  This  field consists of the actual data,

converted to two ascii characters,  high

digit first.  There are no data bytes in

the End of File record.

Checksum Field       -  The  checksum  field is the 8 bit binary

sum of the record length field, the load

address  field,  the  record type field,

and the data field.  This  sum  is  then

negated  (2's  complement) and converted

to  two  ascii  characters,  high  digit

first.

THE LINKER                                             Page 2-12

MOTOROLA S1-S9 OUTPUT FORMAT

2.8  MOTORLA S1-S9 OUTPUT FORMAT

Record Type Field    -  This  field  signifies  the  start  of a

record and  identifies  the  the  record

type as follows:

Ascii S1 - Data Record

Ascii S9 - End of File Record

Record Length Field  -  This  field  specifies the record length

which includes the  address,  data,  and

checksum   fields.   The  8  bit  record

length value is converted to  two  ascii

characters, high digit first.

Load Address Field   -  This  field  consists  of the four ascii

characters which result from  converting

the  the  binary value of the address in

which to begin loading this record.  The

order is as follows:

High digit of high byte of address.

Low digit of high byte of address.

High digit of low byte of address.

Low digit of low byte of address.

In an End of File record this field con-

sists of either four ascii zeros or  the

program  entry  address.   Currently the

entry address option is not supported.

Data Field           -  This  field consists of the actual data,

converted to two ascii characters,  high

digit first.  There are no data bytes in

the End of File record.

Checksum Field       -  The  checksum  field is the 8 bit binary

sum of the record length field, the load

address field, and the data field.  This

sum is then  complemented  (1's  comple-

ment)   and   converted   to  two  ascii

characters, high digit first.

CHAPTER 3

BUILDING ASXXXX AND ASLINK

The assemblers and linker have been successfully compiled us-

ing  the  DECUS C  (PDP-11)  compiler  (patch  level   9)   with

RT-11/TSX+,  GCC  2.7.2 with Linux, Symantec C/C++ V6.1/V7.2 and

DJGPP (GCC 2.8.2) with MS-DOS and Windows 3.x/95/98.

The  device  specific  header  file  (i.e.  m6800.h, m6801.h,

etc.) contains the DECUS C 'BUILD' directives for  generating  a

command  file to compile, assemble, and link the necessary files

to prepare an executable image for a  particular  assembler.   A

Linux makefile and DJGPP makefile are available to build all the

assemblers and the linker.

3.1  BUILDING AN ASSEMBLER

The  building  of  a typical assembler (6809 for example) re-

quires the following files:

1.  m6809.h

2.  m09ext.c

3.  m09mch.c

4.  m09adr.c

5.  m09pst.c

6.  asxxxx.h

7.  asmain.c

8.  aslex.c

9.  assym.c

10.  assubr.c

11.  asexpr.c

12.  asdata.c

13.  aslist.c

14.  asout.c

The  first  five  files are the 6809 processor dependent sec-

tions which contain the following:

BUILDING ASXXXX AND ASLINK                              PAGE 3-2

BUILDING AN ASSEMBLER

1.  m6809.h -  header  file containing the machine specific

definitions of constants,  variables,  structures,  and

types

2.  m09ext -  device  description, byte order, and file ex-

tension information

3.  m09pst -  a  table of the assembler general directives,

special device directives, and assembler mnemonics with

associated operation codes

4.  m09mch / m09adr -  machine specific code for processing

the device mnemonics,  addressing  modes,  and  special

directives

The  remaining nine files provide the device independent sec-

tions which handle the  details  of  file  input/output,  symbol

table  generation,  program/data areas, expression analysis, and

assembler directive processing.

3.2  BUILDING ASLINK

The building of the linker requires the following files:

1.  aslink.h

2.  lkmain.c

3.  lklex.c

4.  lkarea.c

5.  lkhead.c

6.  lksym.c

7.  lkeval.c

8.  lkdata.c

9.  lklist.c

10.  lkrloc.c

11.  lklibr.c

12.  lks19.c

13.  lkihx.c

APPENDIX A

AS6800 ASSEMBLER

A.1  6800 REGISTER SET

The following is a list of the 6800 registers used by AS6800:

a,b     -       8-bit accumulators

x       -       index register

A.2  6800 INSTRUCTION SET

The following tables list all 6800/6802/6808 mnemonics recog-

nized by the AS6800 assembler.  The designation [] refers  to  a

required addressing mode argument.  The following list specifies

the format for each addressing mode supported by AS6800:

#data           immediate data

byte or word data

*dir            direct page addressing

(see .setdp directive)

0 <= dir <= 255

,x              register indirect addressing

zero offset

offset,x        register indirect addressing

0 <= offset <= 255

ext             extended addressing

label           branch label

The  terms  data, dir, offset, ext, and label may all be expres-

sions.

AS6800 ASSEMBLER                                        PAGE A-2

6800 INSTRUCTION SET

Note  that  not all addressing modes are valid with every in-

struction, refer to the 6800 technical data for valid modes.

A.2.1  Inherent Instructions

aba                     cba

clc                     cli

clv                     daa

des                     dex

ins                     inx

nop                     rti

rts                     sba

sec                     sei

sev                     swi

tab                     tap

tba                     tpa

tsx                     txs

wai

psha                    pshb

psh a                   psh b

pula                    pulb

pul a                   pul b

A.2.2  Branch Instructions

bra     label           bhi     label

bls     label           bcc     label

bhs     label           bcs     label

blo     label           bne     label

beq     label           bvc     label

bvs     label           bpl     label

bmi     label           bge     label

blt     label           bgt     label

ble     label           bsr     label

AS6800 ASSEMBLER                                        PAGE A-3

6800 INSTRUCTION SET

A.2.3  Single Operand Instructions

asla                    aslb

asl a                   asl b

asl     []

asra                    asrb

asr a                   asr b

asr     []

clra                    clrb

clr a                   clr b

clr     []

coma                    comb

com a                   com b

com     []

deca                    decb

dec a                   dec b

dec     []

inca                    incb

inc a                   inc b

inc     []

lsla                    lslb

lsl a                   lsl b

lsl     []

lsra                    lsrb

lsr a                   lsr b

lsr     []

nega                    negb

neg a                   neg b

neg     []

rola                    rolb

rol a                   rol b

rol     []

rora                    rorb

ror a                   ror b

ror     []

tsta                    tstb

tst a                   tst b

tst     []

AS6800 ASSEMBLER                                        PAGE A-4

6800 INSTRUCTION SET

A.2.4  Double Operand Instructions

adca    []              adcb    []

adc a   []              adc b   []

adda    []              addb    []

add a   []              add b   []

anda    []              andb    []

and a   []              and b   []

bita    []              bitb    []

bit a   []              bit b   []

cmpa    []              cmpb    []

cmp a   []              cmp b   []

eora    []              eorb    []

eor a   []              eor b   []

ldaa    []              ldab    []

lda a   []              lda b   []

oraa    []              orab    []

ora a   []              ora b   []

sbca    []              sbcb    []

sbc a   []              sbc b   []

staa    []              stab    []

sta a   []              sta b   []

suba    []              subb    []

sub a   []              sub b   []

A.2.5  Jump and Jump to Subroutine Instructions

jmp     []              jsr     []

AS6800 ASSEMBLER                                        PAGE A-5

6800 INSTRUCTION SET

A.2.6  Long Register Instructions

cpx     []

lds     []              sts     []

ldx     []              stx     []

APPENDIX B

AS6801 ASSEMBLER

B.1  .hd6303 DIRECTIVE

Format:

.hd6303

The  .hd6303 directive enables processing of the HD6303 specific

mnemonics not included in  the  6801  instruction  set.   HD6303

mnemonics  encountered  without  the  .hd6303  directive will be

flagged with an 'o' error.

B.2  6801 REGISTER SET

The following is a list of the 6801 registers used by AS6801:

a,b     -       8-bit accumulators

d       -       16-bit accumulator <a:b>

x       -       index register

B.3  6801 INSTRUCTION SET

The  following tables list all 6801/6303 mnemonics recognized

by the AS6801 assembler.  The designation []  refers  to  a  re-

quired  addressing  mode argument.  The following list specifies

the format for each addressing mode supported by AS6801:

#data           immediate data

byte or word data

*dir            direct page addressing

(see .setdp directive)

0 <= dir <= 255

AS6801 ASSEMBLER                                        PAGE B-2

6801 INSTRUCTION SET

,x              register indirect addressing

zero offset

offset,x        register indirect addressing

0 <= offset <= 255

ext             extended addressing

label           branch label

The  terms  data, dir, offset, ext, and label may all be expres-

sions.

Note  that  not all addressing modes are valid with every in-

struction, refer to  the  6801/6303  technical  data  for  valid

modes.

B.3.1  Inherent Instructions

aba             abx

cba             clc

cli             clv

daa             des

dex             ins

inx             mul

nop             rti

rts             sba

sec             sei

sev             swi

tab             tap

tba             tpa

tsx             txs

wai

B.3.2  Branch Instructions

bra     label           brn     label

bhi     label           bls     label

bcc     label           bhs     label

bcs     label           blo     label

bne     label           beq     label

bvc     label           bvs     label

bpl     label           bmi     label

bge     label           blt     label

bgt     label           ble     label

bsr     label

AS6801 ASSEMBLER                                        PAGE B-3

6801 INSTRUCTION SET

B.3.3  Single Operand Instructions

asla            aslb            asld

asl a           asl b           asl d

asl     []

asra            asrb

asr a           asr b

asr     []

clra            clrb

clr a           clr b

clr     []

coma            comb

com a           com b

com     []

deca            decb

dec a           dec b

dec     []

eora            eorb

eor a           eor b

eor     []

inca            incb

inc a           inc b

inc     []

lsla            lslb            lsld

lsl a           lsl b           lsl d

lsl     []

lsra            lsrb            lsrd

lsr a           lsr b           lsr d

lsr     []

nega            negb

neg a           neg b

neg     []

psha            pshb            pshx

psh a           psh b           psh x

pula            pulb            pulx

pul a           pul b           pul x

rola            rolb

rol a           rol b

rol     []

AS6801 ASSEMBLER                                        PAGE B-4

6801 INSTRUCTION SET

rora            rorb

ror a           ror b

ror     []

tsta            tstb

tst a           tst b

tst     []

B.3.4  Double Operand Instructions

adca    []      adcb    []

adc a   []      adc b   []

adda    []      addb    []      addd    []

add a   []      add b   []      add d   []

anda    []      andb    []

and a   []      and b   []

bita    []      bitb    []

bit a   []      bit b   []

cmpa    []      cmpb    []

cmp a   []      cmp b   []

ldaa    []      ldab    []

lda a   []      lda b   []

oraa    []      orab    []

ora a   []      ora b   []

sbca    []      sbcb    []

sbc a   []      sbc b   []

staa    []      stab    []

sta a   []      sta b   []

suba    []      subb    []      subd    []

sub a   []      sub b   []      sub d   []

AS6801 ASSEMBLER                                        PAGE B-5

6801 INSTRUCTION SET

B.3.5  Jump and Jump to Subroutine Instructions

jmp     []      jsr     []

B.3.6  Long Register Instructions

cpx     []      ldd     []

lds     []      ldx     []

std     []      sts     []

stx     []

B.3.7  6303 Specific Instructions

aim     #data, []       eim     #data, []

oim     #data, []       tim     #data, []

xgdx            slp

APPENDIX C

AS6804 ASSEMBLER

Requires the .setdp directive to specify the ram area.

C.1  6804 REGISTER SET

The following is a list of the 6804 registers used by AS6804:

x,y     -       index registers

C.2  6804 INSTRUCTION SET

The  following  tables  list all 6804 mnemonics recognized by

the AS6804 assembler.  The designation [] refers to  a  required

addressing  mode  argument.   The  following  list specifies the

format for each addressing mode supported by AS6804:

#data           immediate data

byte or word data

,x              register indirect addressing

dir             direct addressing

(see .setdp directive)

0 <= dir <= 255

ext             extended addressing

label           branch label

The  terms data, dir, and ext may be expressions.  The label for

the short branchs beq, bne, bcc, and bcs must not be external.

Note  that  not all addressing modes are valid with every in-

struction, refer to the 6804 technical data for valid modes.

AS6804 ASSEMBLER                                        PAGE C-2

6804 INSTRUCTION SET

C.2.1  Inherent Instructions

coma            decx

decy            incx

incy            rola

rti             rts

stop            tax

tay             txa

tya             wait

C.2.2  Branch Instructions

bne     label           beq     label

bcc     label           bcs     label

C.2.3  Single Operand Instructions

add     []

and     []

cmp     []

dec     []

inc     []

lda     []

sta     []

sub     []

C.2.4  Jump and Jump to Subroutine Instructions

jsr     []

jmp     []

C.2.5  Bit Test Instructions

brclr   #data,[],label

brset   #data,[],label

bclr    #label,[]

bset    #label,[]

AS6804 ASSEMBLER                                        PAGE C-3

6804 INSTRUCTION SET

C.2.6  Load Immediate data Instruction

mvi     [],#data

C.2.7  6804 Derived Instructions

asla

bam     label

bap     label

bxmi    label

bxpl    label

bymi    label

bypl    label

clra

clrx

clry

deca

decx

decy

inca

incx

incy

ldxi    #data

ldyi    #data

nop

tax

tay

txa

tya

APPENDIX D

AS6805 ASSEMBLER

D.1  6805 REGISTER SET

The following is a list of the 6805 registers used by AS6805:

a       -       8-bit accumulator

x       -       index register

D.2  6805 INSTRUCTION SET

The  following  tables  list all 6805 mnemonics recognized by

the AS6805 assembler.  The designation [] refers to  a  required

addressing  mode  argument.   The  following  list specifies the

format for each addressing mode supported by AS6805:

#data           immediate data

byte or word data

*dir            direct page addressing

(see .setdp directive)

0 <= dir <= 255

,x              register indirect addressing

zero offset

offset,x        register indirect addressing

0 <= offset <= 255   --- byte mode

256 <= offset <= 65535 --- word mode

(an externally defined offset uses the

word mode)

ext             extended addressing

label           branch label

AS6805 ASSEMBLER                                        PAGE D-2

6805 INSTRUCTION SET

The terms data, dir, offset, and ext may all be expressions.

Note  that  not all addressing modes are valid with every in-

struction, refer to the 6805 technical data for valid modes.

D.2.1  Control Instructions

clc             cli

nop             rsp

rti             rts

sec             sei

stop            swi

tax             txa

wait

D.2.2  Bit Manipulation Instructions

brset   #data,*dir,label

brclr   #data,*dir,label

bset    #data,*dir

bclr    #data,*dir

D.2.3  Branch Instructions

bra     label           brn     label

bhi     label           bls     label

bcc     label           bcs     label

bne     label           beq     label

bhcc    label           bhcs    label

bpl     label           bmi     label

bmc     label           bms     label

bil     label           bih     label

bsr     label

AS6805 ASSEMBLER                                        PAGE D-3

6805 INSTRUCTION SET

D.2.4  Read-Modify-Write Instructions

nega            negx

neg     []

coma            comx

com     []

lsra            lsrx

lsr     []

rora            rorx

ror     []

asra            asrx

asr     []

lsla            lslx

lsl     []

rola            rolx

rol     []

deca            decx

dec     []

inca            incx

inc     []

tsta            tstx

tst     []

clra            clrx

clr     []

D.2.5  Register\Memory Instructions

sub     []              cmp     []

sbc     []              cpx     []

and     []              bit     []

lda     []              sta     []

eor     []              adc     []

ora     []              add     []

ldx     []              stx     []

AS6805 ASSEMBLER                                        PAGE D-4

6805 INSTRUCTION SET

D.2.6  Jump and Jump to Subroutine Instructions

jmp     []              jsr     []

APPENDIX E

AS6808 ASSEMBLER

E.1  68HC08 REGISTER SET

The  following  is  a  list  of  the  68HC08  registers  used by

AS68HC08:

a       -       8-bit accumulator

x       -       index register  <H:X>

s       -       stack pointer

E.2  68HC08 INSTRUCTION SET

The  following tables list all 68HC08 mnemonics recognized by

the AS6808 assembler.  The designation [] refers to  a  required

addressing  mode  argument.   The  following  list specifies the

format for each addressing mode supported by AS6808:

#data           immediate data

byte or word data

*dir            direct page addressing

(see .setdp directive)

0 <= dir <= 255

,x              register indexed addressing

zero offset

offset,x        register indexed addressing

0 <= offset <= 255   --- byte mode

256 <= offset <= 65535 --- word mode

(an externally defined offset uses the

word mode)

,x+             register indexed addressing

zero offset with post increment

AS6808 ASSEMBLER                                        PAGE E-2

68HC08 INSTRUCTION SET

offset,x+       register indexed addressing

unsigned byte offset with post increment

offset,s        stack pointer indexed addressing

0 <= offset <= 255   --- byte mode

256 <= offset <= 65535 --- word mode

(an externally defined offset uses the

word mode)

ext             extended addressing

label           branch label

The terms data, dir, offset, and ext may all be expressions.

Note  that  not all addressing modes are valid with every in-

struction, refer to the 68HC08 technical data for valid modes.

E.2.1  Control Instructions

clc             cli             daa             div

mul             nop             nsa             psha

pshh            pshx            pula            pulh

pulx            rsp             rti             rts

sec             sei             stop            swi

tap             tax             tpa             tsx

txa             txs             wait

E.2.2  Bit Manipulation Instructions

brset   #data,*dir,label

brclr   #data,*dir,label

bset    #data,*dir

bclr    #data,*dir

AS6808 ASSEMBLER                                        PAGE E-3

68HC08 INSTRUCTION SET

E.2.3  Branch Instructions

bra     label           brn     label

bhi     label           bls     label

bcc     label           bcs     label

bne     label           beq     label

bhcc    label           bhcs    label

bpl     label           bmi     label

bmc     label           bms     label

bil     label           bih     label

bsr     label           bge     label

blt     label           bgt     label

ble     label

E.2.4  Complex Branch Instructions

cbeqa   [],label

cbeqx   [],label

cbeq    [],label

dbnza   label

dbnzx   label

dbnz    [],label

AS6808 ASSEMBLER                                        PAGE E-4

68HC08 INSTRUCTION SET

E.2.5  Read-Modify-Write Instructions

nega                    negx

neg     []

coma                    comx

com     []

lsra                    lsrx

lsr     []

rora                    rorx

ror     []

asra                    asrx

asr     []

asla                    aslx

asl     []

lsla                    lslx

lsl     []

rola                    rolx

rol     []

deca                    decx

dec     []

inca                    incx

inc     []

tsta                    tstx

tst     []

clra                    clrx

clr     []              clrh

aix     #data

ais     #data

AS6808 ASSEMBLER                                        PAGE E-5

68HC08 INSTRUCTION SET

E.2.6  Register\Memory Instructions

sub     []              cmp     []

sbc     []              cpx     []

and     []              bit     []

lda     []              sta     []

eor     []              adc     []

ora     []              add     []

ldx     []              stx     []

E.2.7  Double Operand Move Instruction

mov     [],[]

E.2.8  16-Bit <H:X> Index Register Instructions

cphx    []

ldhx    []

sthx    []

E.2.9  Jump and Jump to Subroutine Instructions

jmp     []              jsr     []

APPENDIX F

AS6809 ASSEMBLER

F.1  6809 REGISTER SET

The following is a list of the 6809 registers used by AS6809:

a,b     -       8-bit accumulators

d       -       16-bit accumulator <a:b>

x,y     -       index registers

s,u     -       stack pointers

pc      -       program counter

cc      -       condition code

dp      -       direct page

F.2  6809 INSTRUCTION SET

The  following  tables  list all 6809 mnemonics recognized by

the AS6809 assembler.  The designation [] refers to  a  required

addressing  mode  argument.   The  following  list specifies the

format for each addressing mode supported by AS6809:

#data           immediate data

byte or word data

*dir            direct page addressing

(see .setdp directive)

0 <= dir <= 255

label           branch label

r,r1,r2         registers

cc,a,b,d,dp,x,y,s,u,pc

,-x     ,--x    register indexed

autodecrement

AS6809 ASSEMBLER                                        PAGE F-2

6809 INSTRUCTION SET

,x+     ,x++    register indexed

autoincrement

,x              register indexed addressing

zero offset

offset,x        register indexed addressing

-16 <= offset <= 15    ---  5-bit

-128 <= offset <= -17   ---  8-bit

16 <= offset <= 127   ---  8-bit

-32768 <= offset <= -129  --- 16-bit

128 <= offset <= 32767 --- 16-bit

(external definition of offset

uses 16-bit mode)

a,x             accumulator offset indexed addressing

ext             extended addressing

ext,pc          pc addressing ( pc <- pc + ext )

ext,pcr         pc relative addressing

[,--x]          register indexed indirect

autodecrement

[,x++]          register indexed indirect

autoincrement

[,x]            register indexed indirect addressing

zero offset

[offset,x]      register indexed indirect addressing

-128 <= offset <= 127   ---  8-bit

-32768 <= offset <= -129  --- 16-bit

128 <= offset <= 32767 --- 16-bit

(external definition of offset

uses 16-bit mode)

[a,x]           accumulator offset indexed

indirect addressing

[ext]           extended indirect addressing

[ext,pc]        pc indirect addressing

( [pc <- pc + ext] )

[ext,pcr]       pc relative indirect addressing

The  terms  data, dir, label, offset, and ext may all be expres-

sions.

AS6809 ASSEMBLER                                        PAGE F-3

6809 INSTRUCTION SET

Note  that  not all addressing modes are valid with every in-

struction, refer to the 6809 technical data for valid modes.

F.2.1  Inherent Instructions

abx             daa

mul             nop

rti             rts

sex             swi

swi1            swi2

swi3            sync

F.2.2  Short Branch Instructions

bcc     label           bcs     label

beq     label           bge     label

bgt     label           bhi     label

bhis    label           bhs     label

ble     label           blo     label

blos    label           bls     label

blt     label           bmi     label

bne     label           bpl     label

bra     label           brn     label

bvc     label           bvs     label

bsr     label

F.2.3  Long Branch Instructions

lbcc    label           lbcs    label

lbeq    label           lbge    label

lbgt    label           lbhi    label

lbhis   label           lbhs    label

lble    label           lblo    label

lblos   label           lbls    label

lblt    label           lbmi    label

lbne    label           lbpl    label

lbra    label           lbrn    label

lbvc    label           lbvs    label

lbsr    label

AS6809 ASSEMBLER                                        PAGE F-4

6809 INSTRUCTION SET

F.2.4  Single Operand Instructions

asla            aslb

asl     []

asra            asrb

asr     []

clra            clrb

clr     []

coma            comb

com     []

deca            decb

dec     []

inca            incb

inc     []

lsla            lslb

lsl     []

lsra            lsrb

lsr     []

nega            negb

neg     []

rola            rolb

rol     []

rora            rorb

ror     []

tsta            tstb

tst     []

AS6809 ASSEMBLER                                        PAGE F-5

6809 INSTRUCTION SET

F.2.5  Double Operand Instructions

adca    []              adcb    []

adda    []              addb    []

anda    []              andb    []

bita    []              bitb    []

cmpa    []              cmpb    []

eora    []              eorb    []

lda     []              ldb     []

ora     []              orb     []

sbca    []              sbcb    []

sta     []              stb     []

suba    []              subb    []

F.2.6  D-register Instructions

addd    []              subd    []

cmpd    []              ldd     []

std     []

F.2.7  Index/Stack Register Instructions

cmps    []              cmpu    []

cmpx    []              cmpy    []

lds     []              ldu     []

ldx     []              ldy     []

leas    []              leau    []

leax    []              leay    []

sts     []              stu     []

stx     []              sty     []

pshs    r               pshu    r

puls    r               pulu    r

AS6809 ASSEMBLER                                        PAGE F-6

6809 INSTRUCTION SET

F.2.8  Jump and Jump to Subroutine Instructions

jmp     []              jsr     []

F.2.9  Register - Register Instructions

exg     r1,r2           tfr     r1,r2

F.2.10  Condition Code Register Instructions

andcc   #data           orcc    #data

cwai    #data

F.2.11  6800 Compatibility Instructions

aba             cba

clc             cli

clv             des

dex             ins

inx

ldaa    []      ldab    []

oraa    []      orab    []

psha            pshb

pula            pulb

sba             sec

sei             sev

staa    []      stab    []

tab             tap

tba             tpa

tsx             txs

wai

APPENDIX G

AS6811 ASSEMBLER

G.1  68HC11 REGISTER SET

The following is a list of the 68HC11 registers used by AS6811:

a,b     -       8-bit accumulators

d       -       16-bit accumulator <a:b>

x,y     -       index registers

G.2  68HC11 INSTRUCTION SET

The  following tables list all 68HC11 mnemonics recognized by

the AS6811 assembler.  The designation [] refers to  a  required

addressing  mode  argument.   The  following  list specifies the

format for each addressing mode supported by AS6811:

#data           immediate data

byte or word data

*dir            direct page addressing

(see .setdp directive)

0 <= dir <= 255

,x              register indirect addressing

zero offset

offset,x        register indirect addressing

0 <= offset <= 255

ext             extended addressing

label           branch label

The terms data, dir, offset, and ext may all be expressions.

AS6811 ASSEMBLER                                        PAGE G-2

68HC11 INSTRUCTION SET

Note  that  not all addressing modes are valid with every in-

struction, refer to the 68HC11 technical data for valid modes.

G.2.1  Inherent Instructions

aba             abx

aby             cba

clc             cli

clv             daa

des             dex

dey             fdiv

idiv            ins

inx             iny

mul             nop

rti             rts

sba             sec

sei             sev

stop            swi

tab             tap

tba             tpa

tsx             txs

wai             xgdx

xgdy

psha            pshb

psh a           psh b

pshx            pshy

psh x           psh y

pula            pulb

pul a           pul b

pulx            puly

pul x           pul y

G.2.2  Branch Instructions

bra     label           brn     label

bhi     label           bls     label

bcc     label           bhs     label

bcs     label           blo     label

bne     label           beq     label

bvc     label           bvs     label

bpl     label           bmi     label

bge     label           blt     label

bgt     label           ble     label

bsr     label

AS6811 ASSEMBLER                                        PAGE G-3

68HC11 INSTRUCTION SET

G.2.3  Single Operand Instructions

asla            aslb            asld

asl a           asl b           asl d

asl     []

asra            asrb

asr a           asr b

asr     []

clra            clrb

clr a           clr b

clr     label

coma            comb

com a           com b

com     []

deca            decb

dec a           dec b

dec     []

inca            incb

inc a           inc b

inc     []

lsla            lslb            lsld

lsl a           lsl b           lsl d

lsl     []

lsra            lsrb            lsrd

lsr a           lsr b           lsr d

lsr     []

nega            negb

neg a           neg b

neg     []

rola            rolb

rol a           rol b

rol     []

rora            rorb

ror a           ror b

ror     []

tsta            tstb

tst a           tst b

tst     []

AS6811 ASSEMBLER                                        PAGE G-4

68HC11 INSTRUCTION SET

G.2.4  Double Operand Instructions

adca    []              adcb    []

adc a   []              adc b   []

adda    []      addb    []      addd    []

add a   []      add b   []      add d   []

anda    []              andb    []

and a   []              and b   []

bita    []              bitb    []

bit a   []              bit b   []

cmpa    []              cmpb    []

cmp a   []              cmp b   []

eora    []              eorb    []

eor a   []              eor b   []

ldaa    []              ldab    []

lda a   []              lda b   []

oraa    []              orab    []

ora a   []              ora b   []

sbca    []              sbcb    []

sbc a   []              sbc b   []

staa    []              stab    []

sta a   []              sta b   []

suba    []      subb    []      subd    []

sub a   []      sub b   []      sub d   []

G.2.5  Bit Manupulation Instructions

bclr    [],#data

bset    [],#data

brclr   [],#data,label

brset   [],#data,label

AS6811 ASSEMBLER                                        PAGE G-5

68HC11 INSTRUCTION SET

G.2.6  Jump and Jump to Subroutine Instructions

jmp     []              jsr     []

G.2.7  Long Register Instructions

cpx     []              cpy     []

ldd     []              lds     []

ldx     []              ldy     []

std     []              sts     []

stx     []              sty     []

APPENDIX H

AS6812 ASSEMBLER

H.1  68HC12 REGISTER SET

The following is a list of the 68HC12 registers used by AS6812:

a,b     -       8-bit accumulators

d       -       16-bit accumulator <a:b>

x,y     -       index registers

sp,s    -       stack pointer

pc      -       program counter

ccr,cc  -       condition code register

H.2  68HC12 INSTRUCTION SET

The  following tables list all 68HC12 mnemonics recognized by

the AS6812 assembler.  The designation [] refers to  a  required

addressing  mode  argument.   The  following  list specifies the

format for each addressing mode supported by AS6812:

#data           immediate data

byte or word data

ext             extended addressing

pg              memory page number

*dir            direct page addressing

(see .setdp directive)

0 <= dir <= 255

label           branch label

r,r1,r2         registers

ccr,a,b,d,x,y,sp,pc

AS6812 ASSEMBLER                                        PAGE H-2

68HC12 INSTRUCTION SET

-x      x-      register indexed, pre or

,-x     ,x-     post autodecrement by 1

n,-x    n,x-    register indexed, pre or

post autodecrement by 1 - 8

+x      x+      register indexed, pre or

,+x     ,x+     post autoincrement by 1

n,+x    n,x+    register indexed, pre or

post autoincrement by 1 - 8

offset,x        register indexed addressing

-16 <= offset <= 15    ---  5-bit

-256 <= offset <= -17   ---  9-bit

16 <= offset <= 255   ---  9-bit

-32768 <= offset <= -257  --- 16-bit

256 <= offset <= 32767 --- 16-bit

(external definition of offset

uses 16-bit mode)

[offset,x]      register indexed indirect addressing

-32768 <= offset <= 32767 --- 16-bit

[,x]            register indexed indirect addressing

zero offset

a,x             accumulator offset indexed addressing

[d,x]           d accumulator offset indexed

indirect addressing

The  terms  data, dir, label, offset, and ext may all be expres-

sions.

Note  that  not all addressing modes are valid with every in-

struction, refer to the 68HC12 technical data for valid modes.

AS6812 ASSEMBLER                                        PAGE H-3

68HC12 INSTRUCTION SET

H.2.1  Inherent Instructions

aba             bgnd            cba

daa             dex             dey

ediv            edivs           emul

emuls           fdiv            idiv

idivs           inx             iny

mem             mul             nop

psha            pshb            pshc

pshd            pshx            pshy

pula            pulb            pulc

puld            pulx            puly

rev             revw            rtc

rti             rts             sba

stop            swi             tab

tba             wai             wav

wavr

H.2.2  Short Branch Instructions

bcc     label           bcs     label

beq     label           bge     label

bgt     label           bhi     label

bhis    label           bhs     label

ble     label           blo     label

blos    label           bls     label

blt     label           bmi     label

bne     label           bpl     label

bra     label           brn     label

bvc     label           bvs     label

bsr     label

H.2.3  Long Branch Instructions

lbcc    label           lbcs    label

lbeq    label           lbge    label

lbgt    label           lbhi    label

lbhis   label           lbhs    label

lble    label           lblo    label

lblos   label           lbls    label

lblt    label           lbmi    label

lbne    label           lbpl    label

lbra    label           lbrn    label

lbvc    label           lbvs    label

AS6812 ASSEMBLER                                        PAGE H-4

68HC12 INSTRUCTION SET

H.2.4  Branch on Decrement, Test, or Increment

dbeq    r,label         dbne    r,label

ibeq    r,label         ibne    r,label

tbeq    r,label         tbne    r,label

H.2.5  Bit Clear and Set Instructions

bclr    [],#data

bset    [],#data

H.2.6  Branch on Bit Clear or Set

brclr   [],#data,label

brset   [],#data,label

AS6812 ASSEMBLER                                        PAGE H-5

68HC12 INSTRUCTION SET

H.2.7  Single Operand Instructions

asla            aslb

asl     []

asra            asrb

asr     []

clra            clrb

clr     []

coma            comb

com     []

deca            decb

dec     []

inca            incb

inc     []

lsla            lslb

lsl     []

lsra            lsrb

lsr     []

nega            negb

neg     []

rola            rolb

rol     []

rora            rorb

ror     []

tsta            tstb

tst     []

AS6812 ASSEMBLER                                        PAGE H-6

68HC12 INSTRUCTION SET

H.2.8  Double Operand Instructions

adca    []              adcb    []

adda    []              addb    []

anda    []              andb    []

bita    []              bitb    []

cmpa    []              cmpb    []

eora    []              eorb    []

ldaa    []      <=>     lda     []

ldab    []      <=>     ldb     []

oraa    []      <=>     ora     []

orab    []      <=>     orb     []

sbca    []              sbcb    []

staa    []      <=>     sta     []

stab    []      <=>     stb     []

suba    []              subb    []

H.2.9  Move Instructions

movb    [],[]           movw    [],[]

H.2.10  D-register Instructions

addd    []              subd    []

cpd     []      <=>     cmpd    []

ldd     []              std     []

AS6812 ASSEMBLER                                        PAGE H-7

68HC12 INSTRUCTION SET

H.2.11  Index/Stack Register Instructions

cps     []      <=>     cmps    []

cpx     []      <=>     cmpx    []

cpy     []      <=>     cmpy    []

lds     []

ldx     []              ldy     []

leas    []

leax    []              leay    []

sts     []

stx     []              sty     []

H.2.12  Jump and Jump/Call to Subroutine Instructions

call    [],pg

jmp     []              jsr     []

H.2.13  Other Special Instructions

emacs   []

emaxd   []              emaxm   []

emind   []              eminm   []

etbl    []

maxa    []              maxm    []

mina    []              minm    []

tbl     []              trap    #data

H.2.14  Register - Register Instructions

exg     r1,r2           sex     r1,r2

tfr     r1,r2

H.2.15  Condition Code Register Instructions

andcc   #data           orcc    #data

AS6812 ASSEMBLER                                        PAGE H-8

68HC12 INSTRUCTION SET

H.2.16  M68HC11 Compatibility Mode Instructions

abx             aby             clc

cli             clv             des

ins             sec             sei

sev             tap             tpa

tsx             tsy             txs

tys             xgdx            xgdy

APPENDIX I

AS6816 ASSEMBLER

I.1  68HC16 REGISTER SET

The following is a list of the 68HC16 registers used by AS6816:

a,b     -       8-bit accumulators

d       -       16-bit accumulator <a:b>

e       -       16-bit accumulator

x,y,z   -       index registers

k       -       address extension register

s       -       stack pointer

ccr     -       condition code

I.2  68HC16 INSTRUCTION SET

The  following tables list all 68HC16 mnemonics recognized by

the AS6816 assembler.  The designation [] refers to  a  required

addressing  mode  argument.   The  following  list specifies the

format for each addressing mode supported by AS6816:

#data           immediate data

byte or word data

#xo,#yo         local immediate data (mac / rmac)

label           branch label

r               register

ccr,a,b,d,e,x,y,z,s

,x              zero offset register indexed addressing

,x8

,x16

offset,x        register indexed addressing

AS6816 ASSEMBLER                                        PAGE I-2

68HC16 INSTRUCTION SET

0 <= offset <= 255   ---  8-bit

-32768 <= offset <= -1    --- 16-bit

256 <= offset <= 32767 --- 16-bit

(external definition of offset

uses 16-bit mode)

offset,x8       unsigned 8-bit offset indexed addressing

offset,x16      signed 16-bit offset indexed addressing

e,x             accumulator offset indexed addressing

ext             extended addressing

bank            64K bank number (jmp / jsr)

The  terms data, label, offset, bank, and ext may all be expres-

sions.

Note  that  not all addressing modes are valid with every in-

struction, refer to the 6816 technical data for valid modes.

I.2.1  Inherent Instructions

aba             abx             aby             abz

ace             aced            ade             adx

ady             adz             aex             aey

aez             bgnd            cba             daa

ediv            edivs           emul            emuls

fdiv            fmuls           idiv            ldhi

lpstop          mul             nop             psha

pshb            pshmac          pula            pulb

pulmac          rtr             rts             sba

sde             sted            swi             sxt

tab             tap             tba             tbek

tbsk            tbxk            tbyk            tbzk

tde             tdmsk           tdp             ted

tedm            tekb            tem             tmer

tmet            tmxed           tpa             tpd

tskb            tsx             tsy             tsz

txkb            txs             txy             txz

tykb            tys             tyx             tyz

tzkb            tzs             tzx             tzy

wai             xgab            xgde            xgdx

xgdy            xgdz            xgex            xgey

xgez

AS6816 ASSEMBLER                                        PAGE I-3

68HC16 INSTRUCTION SET

I.2.2  Push/Pull Multiple Register Instructions

pshm    r,...           pulm    r,...

I.2.3  Short Branch Instructions

bcc     label           bcs     label

beq     label           bge     label

bgt     label           bhi     label

bhis    label           bhs     label

ble     label           blo     label

blos    label           bls     label

blt     label           bmi     label

bne     label           bpl     label

bra     label           brn     label

bvc     label           bvs     label

bsr     label

I.2.4  Long Branch Instructions

lbcc    label           lbcs    label

lbeq    label           lbge    label

lbgt    label           lbhi    label

lbhis   label           lbhs    label

lble    label           lblo    label

lblos   label           lbls    label

lblt    label           lbmi    label

lbne    label           lbpl    label

lbra    label           lbrn    label

lbvc    label           lbvs    label

lbsr    label

I.2.5  Bit Manipulation Instructions

bclr    [],#data

bset    [],#data

brclr   [],#data,label

brset   [],#data,label

AS6816 ASSEMBLER                                        PAGE I-4

68HC16 INSTRUCTION SET

I.2.6  Single Operand Instructions

asla                    aslb

asld                    asle

aslm

asl     []              aslw    []

asra                    asrb

asrd                    asre

asrm

asr     []              asrw    []

clra                    clrb

clrd                    clre

clrm

clr     []              clrw    []

coma                    comb

comd                    come

com     []              comw    []

deca                    decb

dec     []              decw    []

inca                    incb

inc     []              incw    []

lsla                    lslb

lsld                    lsle

lslm

lsl     []              lslw    []

lsra                    lsrb

lsrd                    lsre

lsr     []              lsrw    []

nega                    negb

negd                    nege

neg     []              negw    []

rola                    rolb

rold                    role

rol     []              rolw    []

rora                    rorb

rord                    rore

ror     []              rorw    []

tsta                    tstb

tsta                    tste

tst     []              tstw    []

AS6816 ASSEMBLER                                        PAGE I-5

68HC16 INSTRUCTION SET

I.2.7  Double Operand Instructions

adca    []              adcb    []

adcd    []              adce    []

adda    []              addb    []

addd    []              adde    []

anda    []              andb    []

andd    []              ande    []

bita    []              bitb    []

cmpa    []              cmpb    []

cpd     []              cpe     []

eora    []              eorb    []

eord    []              eore    []

ldaa    []              ldab    []

ldd     []              lde     []

oraa    []              orab    []

ord     []              ore     []

sbca    []              sbcb    []

sbcd    []              sbce    []

staa    []              stab    []

std     []              ste     []

suba    []              subb    []

subd    []              sube    []

I.2.8  Index/Stack Register Instructions

cps     []              cpx     []

cpy     []              cpz     []

lds     []              ldx     []

ldy     []              ldz     []

sts     []              stx     []

sty     []              stz     []

AS6816 ASSEMBLER                                        PAGE I-6

68HC16 INSTRUCTION SET

I.2.9  Jump and Jump to Subroutine Instructions

jmp     bank,[]         jsr     bank,[]

I.2.10  Condition Code Register Instructions

andp    #data           orp     #data

I.2.11  Multiply and Accumulate Instructions

mac     #data           rmac    #data

mac     #xo,#yo         rmac    #xo,#yo

APPENDIX J

ASH8 ASSEMBLER

J.1  H8/3XX REGISTER SET

The following is a list of the H8 registers used by ASH8:

r0  -  r7,sp            16-bit accumulators

r0L -  r7L,spL          8-bit accumulators

r0H -  r7H,spH          8-bit accumulators

spL,spH,sp              stack pointers

ccr                     condition code

J.2  H8/3XX INSTRUCTION SET

The  following tables list all H8/3xx mnemonics recognized by

the ASH8 assembler.  The designation [] refers to a required ad-

dressing mode argument.  The following list specifies the format

for each addressing mode supported by ASH8:

#xx:3           immediate data (3  bit)

#xx:8           immediate data (8  bit)

#xx:16          immediate data (16 bit)

*dir            direct page addressing

(see .setdp directive)

0xFF00 <= dir <= 0xFFFF

label           branch label

rn              registers (16 bit)

r0-r7,sp

rnB             registers (8 bit)

r0H-r7H,r0L-r7L,spH,spL

ASH8 ASSEMBLER                                          PAGE J-2

H8/3XX INSTRUCTION SET

ccr             condition code register

@rn             register indirect

@-rn            register indirect (auto pre-decrement)

@rn+            register indirect (auto post-increment)

@[offset,rn]    register indirect, 16-bit displacement

@@offset        memory indirect, (8-bit address)

ext             extended addressing (16-bit)

The  terms  data, dir, label, offset, and ext may all be expres-

sions.

Note  that  not all addressing modes are valid with every in-

struction, refer to the H8/3xx technical data for valid modes.

J.2.1  Inherent Instructions

eepmov

nop

sleep

rte

rts

J.2.2  Branch Instructions

bcc     label                   bcs     label

beq     label                   bf      label

bge     label                   bgt     label

bhi     label                   bhis    label

bhs     label                   ble     label

blo     label                   blos    label

bls     label                   blt     label

bmi     label                   bne     label

bpl     label                   bra     label

brn     label                   bt      label

bvc     label                   bvs     label

bsr     label

ASH8 ASSEMBLER                                          PAGE J-3

H8/3XX INSTRUCTION SET

J.2.3  Single Operand Instructions

Free Form

daa     rnB                     das     rnB

dec     rnB                     inc     rnB

neg     rnB                     not     rnB

rotxl   rnB                     rotxr   rnB

rotl    rnB                     rotr    rnB

shal    rnB                     shar    rnB

shll    rnB                     shlr    rnB

push    rn                      pop     rn

Byte / Word Form

daa.b   rnB                     das.b   rnB

dec.b   rnB                     inc.b   rnB

neg.b   rnB                     not.b   rnB

rotxl.b rnB                     rotxr.b rnB

rotl.b  rnB                     rotr.b  rnB

shal.b  rnB                     shar.b  rnB

shll.b  rnB                     shlr.b  rnB

push.w  rn                      pop.w   rn

ASH8 ASSEMBLER                                          PAGE J-4

H8/3XX INSTRUCTION SET

J.2.4  Double Operand Instructions

Free Form

add     rnB,rnB                 add     #xx:8,rnB

add     rn,rn

adds    #1,rn                   adds    #2,rn

addx    rnB,rnB                 addx    #xx:8,rnB

cmp     rnB,rnB                 cmp     #xx:8,rnB

cmp     rn,rn

sub     rnB,rnB

sub     rn,rn

subs    #1,rn                   subs    #2,rn

subx    rnB,rnB                 subx    #xx:8,rnB

and     rnB,rnB                 and     #xx:8,rnB

and     #xx:8,ccr

or      rnB,rnB                 or      #xx:8,rnB

or      #xx:8,ccr

xor     rnB,rnB                 xor     #xx:8,rnB

xor     #xx:8,ccr

Byte / Word Form

add.b   rnB,rnB                 add.b   #xx:8,rnB

add.w   rn,rn

cmp.b   rnB,rnB                 cmp.b   #xx:8,rnB

cmp.w   rn,rn

sub.b   rnB,rnB

sub.w   rn,rn

addx.b  rnB,rnB                 addx.b  #xx:8,rnB

and.b   rnB,rnB                 and.b   #xx:8,rnB

and.b   #xx:8,ccr

or.b    rnB,rnB                 or.b    #xx:8,rnB

or.b    #xx:8,ccr

subx.b  rnB,rnB                 subx.b  #xx:8,rnB

xor.b   rnB,rnB                 xor.b   #xx:8,rnB

xor.b   #xx:8,ccr

ASH8 ASSEMBLER                                          PAGE J-5

H8/3XX INSTRUCTION SET

J.2.5  Mov Instructions

Free Form

mov     rnB,rnB                 mov     rn,rn

mov     #xx:8,rnB               mov     #xx:16,rn

mov     @rn,rnB                 mov     @rn,rn

mov     @[offset,rn],rnB        mov     @[offset,rn],rn

mov     @rn+,rnB                mov     @rn+,rn

mov     @dir,rnB

mov     dir,rnB

mov     *@dir,rnB

mov     *dir,rnB

mov     @label,rnB              mov     @label,rn

mov     label,rnB               mov     label,rn

mov     rnB,@rn                 mov     rn,@rn

mov     rnB,@[offset,rn]        mov     rn,@[offset,rn]

mov     rnB,@-rn                mov     rn,@-rn

mov     rnB,@dir

mov     rnB,dir

mov     rnB,*@dir

mov     rnB,*dir

mov     rnB,@label              mov     rn,@label

mov     rnB,label               mov     rn,label

Byte / Word Form

mov.b   rnB,rnB                 mov.w   rn,rn

mov.b   #xx:8,rnB               mov.w   #xx:16,rn

mov.b   @rn,rnB                 mov.w   @rn,rn

mov.b   @[offset,rn],rnB        mov.w   @[offset,rn],rn

mov.b   @rn+,rnB                mov.w   @rn+,rn

mov.b   @dir,rnB

mov.b   dir,rnB

mov.b   *@dir,rnB

mov.b   *dir,rnB

mov.b   @label,rnB              mov.w   @label,rn

mov.b   label,rnB               mov.w   label,rn

mov.b   rnB,@rn                 mov.w   rn,@rn

mov.b   rnB,@[offset,rn]        mov.w   rn,@[offset,rn]

mov.b   rnB,@-rn                mov.w   rn,@-rn

mov.b   rnB,@dir

mov.b   rnB,dir

mov.b   rnB,*@dir

mov.b   rnB,*dir

mov.b   rnB,@label              mov.w   rn,@label

mov.b   rnB,label               mov.w   rn,label

ASH8 ASSEMBLER                                          PAGE J-6

H8/3XX INSTRUCTION SET

J.2.6  Bit Manipulation Instructions

bld     #xx:3,rnB               bld     #xx:3,@rn

bld     #xx:3,@dir              bld     #xx:3,dir

bld     #xx:3,*@dir             bld     #xx:3,*dir

bild    #xx:3,rnB               bild    #xx:3,@rn

bild    #xx:3,@dir              bild    #xx:3,dir

bild    #xx:3,*@dir             bild    #xx:3,*dir

bst     #xx:3,rnB               bst     #xx:3,@rn

bst     #xx:3,@dir              bst     #xx:3,dir

bst     #xx:3,*@dir             bst     #xx:3,*dir

bist    #xx:3,rnB               bist    #xx:3,@rn

bist    #xx:3,@dir              bist    #xx:3,dir

bist    #xx:3,*@dir             bist    #xx:3,*dir

band    #xx:3,rnB               band    #xx:3,@rn

band    #xx:3,@dir              band    #xx:3,dir

band    #xx:3,*@dir             band    #xx:3,*dir

biand   #xx:3,rnB               biand   #xx:3,@rn

biand   #xx:3,@dir              biand   #xx:3,dir

biand   #xx:3,*@dir             biand   #xx:3,*dir

bor     #xx:3,rnB               bor     #xx:3,@rn

bor     #xx:3,@dir              bor     #xx:3,dir

bor     #xx:3,*@dir             bor     #xx:3,*dir

bior    #xx:3,rnB               bior    #xx:3,@rn

bior    #xx:3,@dir              bior    #xx:3,dir

bior    #xx:3,*@dir             bior    #xx:3,*dir

bxor    #xx:3,rnB               bxor    #xx:3,@rn

bxor    #xx:3,@dir              bxor    #xx:3,dir

bxor    #xx:3,*@dir             bxor    #xx:3,*dir

bixor   #xx:3,rnB               bixor   #xx:3,@rn

bixor   #xx:3,@dir              bixor   #xx:3,dir

bixor   #xx:3,*@dir             bixor   #xx:3,*dir

ASH8 ASSEMBLER                                          PAGE J-7

H8/3XX INSTRUCTION SET

J.2.7  Extended Bit Manipulation Instructions

bset    #xx:3,rnB               bset    #xx:3,@rn

bset    #xx:3,@dir              bset    #xx:3,dir

bset    #xx:3,*@dir             bset    #xx:3,*dir

bset    rnB,rnB                 bset    rnB,@rn

bset    rnB,@dir                bset    rnB,dir

bset    rnB,*@dir               bset    rnB,*dir

bclr    #xx:3,rnB               bclr    #xx:3,@rn

bclr    #xx:3,@dir              bclr    #xx:3,dir

bclr    #xx:3,*@dir             bclr    #xx:3,*dir

bclr    rnB,rnB                 bclr    rnB,@rn

bclr    rnB,@dir                bclr    rnB,dir

bclr    rnB,*@dir               bclr    rnB,*dir

bnot    #xx:3,rnB               bnot    #xx:3,@rn

bnot    #xx:3,@dir              bnot    #xx:3,dir

bnot    #xx:3,*@dir             bnot    #xx:3,*dir

bnot    rnB,rnB                 bnot    rnB,@rn

bnot    rnB,@dir                bnot    rnB,dir

bnot    rnB,*@dir               bnot    rnB,*dir

btst    #xx:3,rnB               btst    #xx:3,@rn

btst    #xx:3,@dir              btst    #xx:3,dir

btst    #xx:3,*@dir             btst    #xx:3,*dir

btst    rnB,rnB                 btst    rnB,@rn

btst    rnB,@dir                btst    rnB,dir

btst    rnB,*@dir               btst    rnB,*dir

J.2.8  Condition Code Instructions

andc    #xx:8,ccr               andc    #xx:8

and     #xx:8,ccr               and.b   #xx:8,ccr

ldc     #xx:8,ccr               ldc     #xx:8

ldc     rnB,ccr                 ldc     rnB

orc     #xx:8,ccr               orc     #xx:8

or      #xx:8,ccr               or.b    #xx:8,ccr

xorc    #xx:8,ccr               xorc    #xx:8

xor     #xx:8,ccr               xor.b   #xx:8,ccr

stc     ccr,rnB                 stc     rnB

ASH8 ASSEMBLER                                          PAGE J-8

H8/3XX INSTRUCTION SET

J.2.9  Other Instructions

divxu   rnB,rn                  divxu.b rnB,rn

mulxu   rnB,rn                  mulxu.b rnB,rn

movfpe  @label,rnB              movfpe  label,rnB

movfpe.b  @label,rnB            movfpe.b  label,rnB

movtpe  @label,rnB              movtpe  label,rnB

movtpe.b  @label,rnB            movtpe.b  label,rnB

J.2.10  Jump and Jump to Subroutine Instructions

jmp     @rn                     jmp     @@dir

jmp     @label                  jmp     label

jsr     @rn                     jsr     @@dir

jsr     @label                  jsr     label

APPENDIX K

AS8051 ASSEMBLER

K.1  ACKNOWLEDGMENT

Thanks  to  John  Hartman  for his contribution of the AS8051

cross assembler.

John L. Hartman

jhartman@compuserve.com

K.2  8051 REGISTER SET

The following is a list of the 8051 registers used by AS8051:

a,b             -       8-bit accumulators

r0,r1,r2,r3     -       8-bit registers

r4,r5,r6,r7

dptr            -       data pointer

sp              -       stack pointer

pc              -       program counter

psw             -       status word

c               -       carry (bit in status word)

AS8051 ASSEMBLER                                        PAGE K-2

8051 REGISTER SET

K.3  8051 INSTRUCTION SET

The  following  tables  list all 8051 mnemonics recognized by

the AS8051 assembler.  The following list specifies  the  format

for each addressing mode supported by AS8051:

#data           immediate data

byte or word data

r,r1,r2         register r0,r1,r2,r3,r4,r5,r6, or r7

@r              indirect on register r0 or r1

@dptr           indirect on data pointer

@a+dptr         indirect on accumulator

plus data pointer

@a+pc           indirect on accumulator

plus program counter

addr            direct memory address

bitaddr         bit address

label           call or jump label

The terms data, addr, bitaddr, and label may all be expressions.

Note  that  not all addressing modes are valid with every in-

struction.  Refer to the 8051 technical data for valid modes.

K.3.1  Inherent Instructions

nop

AS8051 ASSEMBLER                                        PAGE K-3

8051 INSTRUCTION SET

K.3.2  Move Instructions

mov     a,#data         mov     a,addr

mov     a,r             mov     a,@r

mov     r,#data         mov     r,addr

mov     r,a

mov     addr,a          mov     addr,#data

mov     addr,r          mov     addr,@r

mov     addr1,addr2     mov     bitaddr,c

mov     @r,#data        mov     @r,addr

mov     @r,a

mov     c,bitaddr

mov     dptr,#data

movc    a,@a+dptr       movc    a,@a+pc

movx    a,@dptr         movx    a,@r

movx    @dptr,a         movx    @r,a

K.3.3  Single Operand Instructions

clr     a               clr     c

clr     bitaddr

cpl     a               cpl     c

cpl     bitaddr

setb    c               setb    bitaddr

da      a

rr      a               rrc     a

rl      a               rlc     a

swap    a

dec     a               dec     r

dec     @r

inc     a               inc     r

inc     dptr            inc     @r

div     ab              mul     ab

pop     addr            push    addr

AS8051 ASSEMBLER                                        PAGE K-4

8051 INSTRUCTION SET

K.3.4  Two Operand Instructions

add     a,#data         add     a,addr

add     a,r             add     a,@r

addc    a,#data         addc    a,addr

addc    a,r             addc    a,@r

subb    a,#data         subb    a,addr

subb    a,r             subb    a,@r

orl     a,#data         orl     a,addr

orl     a,r             orl     a,@r

orl     addr,a          orl     addr,#data

orl     c,bitaddr       orl     c,/bitaddr

anl     a,#data         anl     a,addr

anl     a,r             anl     a,@r

anl     addr,a          anl     addr,#data

anl     c,bitaddr       anl     c,/bitaddr

xrl     a,#data         xrl     a,addr

xrl     a,r             xrl     a,@r

xrl     addr,a          xrl     addr,#data

xrl     c,bitaddr       xrl     c,/bitaddr

xch     a,addr          xch     a,r

xch     a,@r            xchd    a,@r

K.3.5  Call and Return Instructions

acall   label           lcall   label

ret                     reti

in      data

out     data

rst     data

K.3.6  Jump Instructions

ajmp    label

cjne    a,#data,label   cjne    a,addr,label

cjne    r,#data,label   cjne    @r,#data,label

djnz    r,label         djnz    addr,label

jbc     bitadr,label

jb      bitadr,label    jnb     bitadr,label

jc      label           jnc     label

jz      label           jnz     label

jmp     @a+dptr

ljmp    label           sjmp    label

AS8051 ASSEMBLER                                        PAGE K-5

8051 INSTRUCTION SET

K.3.7  Predefined Symbols:  SFR Map

--------- 4 Bytes ----------

----    ----    ----    ----

FC                                          FF

F8                                          FB

F4                                          F7

F0      B                                   F3

EC                                          EF

E8                                          EB

E4                                          E7

E0      ACC                                 E3

DC                                          DF

D8                                          DB

D4                                          D7

D0      PSW                                 D3

CC   [  TL2     TH2                     ]   CF

C8   [  T2CON           RCAP2L  RCAP2H  ]   CB

C4                                          C7

C0                                          C3

BC                                          BF

B8      IP                                  BB

B4                                          B7

B0      P3                                  B3

AC                                          AF

A8      IE                                  AB

A4                                          A7

A0      P2                                  A3

9C                                          9F

98      SCON    SBUF                        9B

94                                          97

90      P1                                  93

8C      TH0     TH1                         8F

88      TCON    TMOD    TL0     TL1         8B

84                              PCON        87

80      P0      SP      DPL     DPH         83

[...] Indicates Resident in 8052, not 8051

AS8051 ASSEMBLER                                        PAGE K-6

8051 INSTRUCTION SET

K.3.8  Predefined Symbols:  SFR Bit Addresses

---------- 4 BITS ----------

----    ----    ----    ----

FC                                          FF

F8                                          FB

F4      B.4     B.5     B.6     B.7         F7

F0      B.0     B.1     B.2     B.3         F3

EC                                          EF

E8                                          EB

E4      ACC.4   ACC.5   ACC.6   ACC.7       E7

E0      ACC.0   ACC.1   ACC.2   ACC.3       E3

DC                                          DF

D8                                          DB

D4      PSW.4   PSW.5   PSW.6   PSW.7       D7

D0      PSW.0   PSW.1   PSW.2   PSW.3       D3

CC   [  T2CON.4 T2CON.5 T2CON.6 T2CON.7 ]   CF

C8   [  T2CON.0 T2CON.1 T2CON.2 T2CON.3 ]   CB

C4                                          C7

C0                                          C3

BC      IP.4    IP.5    IP.6    IP.7        BF

B8      IP.0    IP.1    IP.2    IP.3        BB

B4      P3.4    P3.5    P3.6    P3.7        B7

B0      P3.0    P3.1    P3.2    P3.3        B3

AC      IE.4    IE.5    EI.6    IE.7        AF

A8      IE.0    IE.1    IE.2    IE.3        AB

A4      P2.4    P2.5    P2.6    P2.7        A7

A0      P2.0    P2.1    P2.2    P2.3        A3

9C      SCON.4  SCON.5  SCON.6  SCON.7      9F

98      SCON.0  SCON.1  SCON.2  SCON.3      9B

94      P1.4    P1.5    P1.6    P1.7        97

90      P1.0    P1.1    P1.2    P1.3        93

8C      TCON.4  TCON.5  TCON.6  TCON.7      8F

88      TCON.0  TCON.1  TCON.2  TCON.3      8B

84      P0.4    P0.5    P0.6    P0.7        87

80      P0.0    P0.1    P0.2    P0.3        83

[...] Indicates Resident in 8052, not 8051

AS8051 ASSEMBLER                                        PAGE K-7

8051 INSTRUCTION SET

K.3.9  Predefined Symbols:  Control Bits

---------- 4 BITS ----------

----    ----    ----    ----

FC                                          FF

F8                                          FB

F4                                          F7

F0                                          F3

EC                                          EF

E8                                          EB

E4                                          E7

E0                                          E3

DC                                          DF

D8                                          DB

D4      RS1     F0      AC      CY          D7

D0      P               OV      RS0         D3

CC   [  TLCK    RCLK    EXF2    TF2     ]   CF

C8   [  CPRL2   CT2     TR2     EXEN2   ]   CB

C4                                          C7

C0                                          C3

BC      PS      PT2                         BF

B8      PX0     PT0     PX1     PT1         BB

B4                                          B7

B0      RXD     TXD     INT0    INT1        B3

AC      ES      ET2             EA          AF

A8      EX0     ET0     EX1     ET1         AB

A4                                          A7

A0                                          A3

9C      REN     SM2     SM1     SM0         9F

98      RI      TI      RB8     TB8         9B

94                                          97

90                                          93

8C      TR0     TF0     TR1     TF1         8F

88      IT0     IE0     IT1     IE1         8B

84                                          87

80                                          83

[...] Indicates Resident in 8052, not 8051

APPENDIX L

AS8085 ASSEMBLER

L.1  8085 REGISTER SET

The  following  is  a  list  of  the 8080/8085 registers used by

AS8085:

a,b,c,d,e,h,l   -       8-bit accumulators

m               -       memory through (hl)

sp              -       stack pointer

psw             -       status word

L.2  8085 INSTRUCTION SET

The  following tables list all 8080/8085 mnemonics recognized

by the AS8085  assembler.   The  following  list  specifies  the

format for each addressing mode supported by AS8085:

#data           immediate data

byte or word data

r,r1,r2         register or register pair

psw,a,b,c,d,e,h,l

bc,de,hl,sp,pc

m               memory address using (hl)

addr            direct memory addressing

label           call or jump label

The terms data, m, addr, and label may be expressions.

Note  that  not all addressing modes are valid with every in-

struction, refer to  the  8080/8085  technical  data  for  valid

modes.

AS8085 ASSEMBLER                                        PAGE L-2

8085 INSTRUCTION SET

L.2.1  Inherent Instructions

cma             cmc

daa             di

ei              hlt

nop             pchl

ral             rar

ret             rim

rrc             rlc

sim             sphl

stc             xchg

xthl

L.2.2  Register/Memory/Immediate Instructions

adc     r       adc     m       aci     #data

add     r       add     m       adi     #data

ana     r       ana     m       ani     #data

cmp     r       cmp     m       cpi     #data

ora     r       ora     m       ori     #data

sbb     r       sbb     m       sbi     #data

sub     r       sub     m       sui     #data

xra     r       xra     m       xri     #data

L.2.3  Call and Return Instructions

cc      label           rc

cm      label           rm

cnc     label           rnc

cnz     label           rnz

cp      label           rp

cpe     label           rpe

cpo     label           rpo

cz      label           rz

call    label

L.2.4  Jump Instructions

jc      label

jm      label

jnc     label

jnz     label

jp      label

jpe     label

jpo     label

jz      label

jmp     label

AS8085 ASSEMBLER                                        PAGE L-3

8085 INSTRUCTION SET

L.2.5  Input/Output/Reset Instructions

in      data

out     data

rst     data

L.2.6  Move Instructions

mov     r1,r2

mov     r,m

mov     m,r

mvi     r,#data

mvi     m,#data

L.2.7  Other Instructions

dcr     r               dcr     m

inr     r               inr     m

dad     r               dcx     r

inx     r               ldax    r

pop     r               push    r

stax    r

lda     addr            lhld    addr

shld    addr            sta     addr

lxi     r,#data

APPENDIX M

ASZ80 ASSEMBLER

M.1  .hd64 DIRECTIVE

Format:

.hd64

The  .hd64  directive enables processing of the HD64180 specific

mnemonics not included in  the  Z80  instruction  set.   HD64180

mnemonics  encountered  without  the  .hd64  directive  will  be

flagged with an 'o' error.

M.2  Z80 REGISTER SET AND CONDITIONS

The following is a complete list of register designations and

condition mnemonics:

byte registers  -       a,b,c,d,e,h,l,i,r

register pairs  -       af,af',bc,de,hl

word registers  -       pc,sp,ix,iy

C -     carry bit set

M -     sign bit set

NC -    carry bit clear

NZ -    zero bit clear

P -     sign bit clear

PE -    parity even

PO -    parity odd

Z -     zero bit set

ASZ80 ASSEMBLER                                         PAGE M-2

Z80 INSTRUCTION SET

M.3  Z80 INSTRUCTION SET

The  following  tables  list all Z80/HD64180 mnemonics recog-

nized by the ASZ80 assembler.  The designation []  refers  to  a

required addressing mode argument.  The following list specifies

the format for each addressing mode supported by ASZ80:

#data           immediate data

byte or word data

n               byte value

rg              a byte register

a,b,c,d,e,h,l

rp              a register pair

bc,de,hl

(hl)            implied addressing or

register indirect addressing

(label)         direct addressing

offset(ix)      indexed addressing with

an offset

label           call/jmp/jr label

The  terms  data,  dir,  offset, and ext may all be expressions.

The terms dir and offset are not allowed to be  external  refer-

ences.

Note  that  not all addressing modes are valid with every in-

struction, refer to the Z80/HD64180  technical  data  for  valid

modes.

ASZ80 ASSEMBLER                                         PAGE M-3

Z80 INSTRUCTION SET

M.3.1  Inherent Instructions

ccf             cpd

cpdr            cpi

cpir            cpl

daa             di

ei              exx

halt            neg

nop             reti

retn            rla

rlca            rld

rra             rrca

rrd             scf

M.3.2  Implicit Operand Instructions

adc     a,[]            adc     []

add     a,[]            add     []

and     a,[]            and     []

cp      a,[]            cp      []

dec     a,[]            dec     []

inc     a,[]            inc     []

or      a,[]            or      []

rl      a,[]            rl      []

rlc     a,[]            rlc     []

rr      a,[]            rr      []

rrc     a,[]            rrc     []

sbc     a,[]            sbc     []

sla     a,[]            sla     []

sra     a,[]            sra     []

srl     a,[]            srl     []

sub     a,[]            sub     []

xor     a,[]            xor     []

ASZ80 ASSEMBLER                                         PAGE M-4

Z80 INSTRUCTION SET

M.3.3  Load Instruction

ld      rg,[]           ld      [],rg

ld      (bc),a          ld      a,(bc)

ld      (de),a          ld      a,(de)

ld      (label),a       ld      a,(label)

ld      (label),rp      ld      rp,(label)

ld      i,a             ld      r,a

ld      a,i             ld      a,r

ld      sp,hl           ld      sp,ix

ld      sp,iy           ld      rp,#data

ldd                     lddr

ldi                     ldir

M.3.4  Call/Return Instructions

call    C,label         ret     C

call    M,label         ret     M

call    NC,label        ret     NC

call    NZ,label        ret     NZ

call    P,label         ret     P

call    PE,label        ret     PE

call    PO,label        ret     PO

call    Z,label         ret     Z

call    label           ret

M.3.5  Jump and Jump to Subroutine Instructions

jp      C,label         jp      M,label

jp      NC,label        jp      NZ,label

jp      P,label         jp      PE,label

jp      PO,label        jp      Z,label

jp      (hl)            jp      (ix)

jp      (iy)            jp      label

djnz    label

jr      C,label         jr      NC,label

jr      NZ,label        jr      Z,label

jr      label

ASZ80 ASSEMBLER                                         PAGE M-5

Z80 INSTRUCTION SET

M.3.6  Bit Manipulation Instructions

bit     n,[]

res     n,[]

set     n,[]

M.3.7  Interrupt Mode and Reset Instructions

im      n

im      n

im      n

rst     n

M.3.8  Input and Output Instructions

in      a,(n)           in      rg,(c)

ind                     indr

ini                     inir

out     (n),a           out     (c),rg

outd                    otdr

outi                    otir

M.3.9  Register Pair Instructions

add     hl,rp           add     ix,rp

add     iy,rp

adc     hl,rp           sbc     hl,rp

ex      (sp),hl         ex      (sp),ix

ex      (sp),iy

ex      de,hl

ex      af,af'

push    rp              pop     rp

ASZ80 ASSEMBLER                                         PAGE M-6

Z80 INSTRUCTION SET

M.3.10  HD64180 Specific Instructions

in0     rg,(n)

out0    (n),rg

otdm                    otdmr

otim                    otimr

mlt     bc              mlt     de

mlt     hl              mlt     sp

slp

tst     a

tstio   #data

APPENDIX N

AS6500 ASSEMBLER

N.1  ACKNOWLEDGMENT

Thanks  to  Marko  Makela  for his contribution of the AS6500

cross assembler.

Marko Makela

Sillitie 10 A

01480 Vantaa

Finland

Internet: Marko.Makela@Helsinki.Fi

EARN/BitNet: msmakela@finuh

Several  additions and modifications were made to his code to

support the following families of 6500 processors:

(1)     650X and 651X processor family

(2)     65F11 and 65F12 processor family

(3)     65C00/21 and 65C29 processor family

(4)     65C02, 65C102, and 65C112 processor family

The  instruction  syntax of this cross assembler contains two

peculiarities:  (1) the addressing indirection is denoted by the

square  brackets  []  and (2) the `bbrx' and `bbsx' instructions

are written `bbr0 memory,label'.

AS6500 ASSEMBLER                                        PAGE N-2

6500 REGISTER SET

N.2  6500 REGISTER SET

The following is a list of the 6500 registers used by AS6500:

a       -       8-bit accumulator

x,y     -       index registers

N.3  6500 INSTRUCTION SET

The  following  tables  list all 6500 family mnemonics recog-

nized by the AS6500 assembler.  The designation [] refers  to  a

required addressing mode argument.  The following list specifies

the format for each addressing mode supported by AS6500:

#data           immediate data

byte or word data

*dir            direct page addressing

(see .setdp directive)

0 <= dir <= 255

offset,x        indexed addressing

offset,y        indexed addressing

address = (offset + (x or y))

[offset,x]      pre-indexed indirect addressing

0 <= offset <= 255

address = contents of location

(offset + (x or y)) mod 256

[offset],y      post-indexed indirect addressing

address = contents of location at offset

plus the value of the y register

[address]       indirect addressing

ext             extended addressing

label           branch label

address,label   direct page memory location

branch label

bbrx and bbsx instruction addressing

The  terms data, dir, offset, address, ext, and label may all be

expressions.

Note  that  not all addressing modes are valid with every in-

struction, refer to the 65xx technical data for valid modes.

AS6500 ASSEMBLER                                        PAGE N-3

6500 INSTRUCTION SET

N.3.1  Processor Specific Directives

The  AS6500  cross  assembler has four (4) processor specific

assembler directives which  define  the  target  65xx  processor

family:

.r6500          Core 650X and 651X family (default)

.r65f11         Core plus 65F11 and 65F12

.r65c00         Core plus 65C00/21 and 65C29

.r65c02         Core plus 65C02, 65C102, and 65C112

N.3.2  65xx Core Inherent Instructions

brk                     clc

cld                     cli

clv                     dex

dey                     inx

iny                     nop

pha                     php

pla                     plp

rti                     rts

sec                     sed

sei                     tax

tay                     tsx

txa                     txs

tya

N.3.3  65xx Core Branch Instructions

bcc     label           bhs     label

bcs     label           blo     label

beq     label           bmi     label

bne     label           bpl     label

bvc     label           bvs     label

N.3.4  65xx Core Single Operand Instructions

asl     []

dec     []

inc     []

lsr     []

rol     []

ror     []

AS6500 ASSEMBLER                                        PAGE N-4

6500 INSTRUCTION SET

N.3.5  65xx Core Double Operand Instructions

adc     []

and     []

bit     []

cmp     []

eor     []

lda     []

ora     []

sbc     []

sta     []

N.3.6  65xx Core Jump and Jump to Subroutine Instructions

jmp     []              jsr     []

N.3.7  65xx Core Miscellaneous X and Y Register Instructions

cpx     []

cpy     []

ldx     []

stx     []

ldy     []

sty     []

AS6500 ASSEMBLER                                        PAGE N-5

6500 INSTRUCTION SET

N.3.8  65F11 and 65F12 Specific Instructions

bbr0    [],label                bbr1    [],label

bbr2    [],label                bbr3    [],label

bbr4    [],label                bbr5    [],label

bbr6    [],label                bbr7    [],label

bbs0    [],label                bbs1    [],label

bbs2    [],label                bbs3    [],label

bbs4    [],label                bbs5    [],label

bbs6    [],label                bbs7    [],label

rmb0    []                      rmb1    []

rmb2    []                      rmb3    []

rmb4    []                      rmb5    []

rmb6    []                      rmb7    []

smb0    []                      smb1    []

smb2    []                      smb3    []

smb4    []                      smb5    []

smb6    []                      smb7    []

N.3.9  65C00/21 and 65C29 Specific Instructions

bbr0    [],label                bbr1    [],label

bbr2    [],label                bbr3    [],label

bbr4    [],label                bbr5    [],label

bbr6    [],label                bbr7    [],label

bbs0    [],label                bbs1    [],label

bbs2    [],label                bbs3    [],label

bbs4    [],label                bbs5    [],label

bbs6    [],label                bbs7    [],label

bra     label

phx                             phy

plx                             ply

rmb0    []                      rmb1    []

rmb2    []                      rmb3    []

rmb4    []                      rmb5    []

rmb6    []                      rmb7    []

smb0    []                      smb1    []

smb2    []                      smb3    []

smb4    []                      smb5    []

smb6    []                      smb7    []

AS6500 ASSEMBLER                                        PAGE N-6

6500 INSTRUCTION SET

N.3.10  65C02, 65C102, and 65C112 Specific Instructions

bbr0    [],label                bbr1    [],label

bbr2    [],label                bbr3    [],label

bbr4    [],label                bbr5    [],label

bbr6    [],label                bbr7    [],label

bbs0    [],label                bbs1    [],label

bbs2    [],label                bbs3    [],label

bbs4    [],label                bbs5    [],label

bbs6    [],label                bbs7    [],label

bra     label

phx                             phy

plx                             ply

rmb0    []                      rmb1    []

rmb2    []                      rmb3    []

rmb4    []                      rmb5    []

rmb6    []                      rmb7    []

smb0    []                      smb1    []

smb2    []                      smb3    []

smb4    []                      smb5    []

smb6    []                      smb7    []

stz     []

trb     []

tsb     []

Additional  addressing  modes for the following core instruc-

tions are also available with the 65C02, 65C102, and 65C112 pro-

cessors.

adc     []                      and     []

cmp     []                      eor     []

lda     []                      ora     []

sbc     []                      sta     []

bit     []                      jmp     []

dec                             inc